1.3 Product front panel

1.4 servo driver installation

1.4.1 The environmental conditions for installation

Since the environment conditions for servo driver installation have the direct influence to the normal function and service life of the servo driver, therefore the environment conditions must be conformed to the following conditions:

- Ambient temperature: 0 to 40℃; ambient humidity: less than 80% (non-condensing).

- Storage temperature: -40 to 50℃; Storage humidity: less than 93% (non-condensing).

- Vibration: less than 0.5G.

- Prevent dropping water, steam, dust powder, cotton fiber or fine metal powder from invasion.

- Prevent oil, salt and corrosive gas from entering it.

- Avoid high temperature, moisture or being wetted due to raining, with the humidity below 80%RH (non-condensing).

- Prohibit the use in the dangerous environment where inflammable or combustible or explosive gas, liquid or solid exists.

- If there are vibration sources nearby (punch press for example) and no way to avoid it, please use absorber or anti-vibration rubber filling piece. If there is disturbance from interferential equipment nearby along the wirings to the servo driver can make the servo driver misoperation. Using noise filters as well as other anti-jamming measure guarantee normal work of the servo driver. However, the noise filter can increase current leakage, therefore should install an insulating transformer in the input terminals of power supply.

- When several driver installments in a control cubicle, for good ventilation please reserve enough space around each driver, install fans to provide effective cooling, keep less than 40℃ for long-term trouble-free service.

1.4.2 The method of installation

- When transporting the product, right lifting tools are required to prevent inverter from   damaging.

- The stacked box of the servo controller are not permitted higher than the limit.

- Please don’t run the servo controller if there is damage or lacking of components.

- Do not place heavy objects on the servo controller.

- Please prevent screw, cable pieces or other conductive objects or oil etc inflammable objects invading the servo controller.

- Do not make it fall or have a strong impact.

- For installing the servo driver, fasten the backboard of the servo driver with M5 screw bolt.

- To provide vertical wind to the heat sink of the servo driver should install ventilating fans in the control cubicle.

- Confirm if the installation location and object could withstand the weight of the servo controller. The servo controller must be installed by wall hooking、indoor room with adequate ventilation, with enough space left between it and the adjacent objects or retaining board (walls) around, as shown in the picture below：

1.5 Servo motor installation

1.5.1 The environmental conditions for installation

- Ambient temperature：0～40℃；Ambient humidity：less than 80 % (non-condensing).

- Storage temperature：-40～50℃；Storage humidity：less than 80% (non-condensing).

- Vibration：less than 0.5G.

- Install the servomotor in well-ventilated place with less moisture and a few dusts.

- Install the servomotor in a place without corrosive liquid, flammable gas, oil vapor, cutting cooling liquid, cutting chips, iron powder and so on.

- Install the servomotor in a place without water vapor and direct sunlight.

1.5.2 The method of installation

- For horizontal installation: In order to prevent water, oil, etc. from entering inside of the servomotor, please put the cable connector downward.

- For vertical installation: if the shaft of the servo motor is in upward direction with a speed reducer, some prevention measure shall be taken against entering inside of the servomotor by oil come from the speed reducer.

- Motor shaft extension should be long enough, or may cause vibration while motor is in running.

- In case of installation or removing the servomotor, please do not hit the servomotor with a hammer, otherwise the shaft and the encoder can be damaged.

1.6 The definition of rotating direction for servomotor

The motor rotating direction description in this handbook is defined as facing the shaft of the servomotor, if the rotating shaft is in counterclockwise direction will be called as positive direction, or in clockwise as reversal direction.

CCW direction                  CW direction

1.7 Servo power standard

| Input power supply | Power supply control | 220V Class:1 Phase AC200V-230V+10%-15%,50/60Hz |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 400V Class:1 Phase AC380V-480V+10%-15%,50/60Hz |  |  |  |  |  |  |  |  |  |
|  | Main power | 220V Class:3 Phase AC200V-230V+10%-15%,50/60Hz |  |  |  |  |  |  |  |  |  |
|  |  | 400V Class:3 Phase AC380V-480V+10%-15%,50/60Hz |  |  |  |  |  |  |  |  |  |
| 200V | 100W | 200W | 400W | 600W | 1KW | 2KW | 3.5KW | 5KW | 7KW | 11KW | 15KW |
| Rated current | 1.1 | 1.5 | 2.7 | 3.2 | 5.8 | 12 | 16 | 28 | 36 | 57 | 77 |
| Maximum current | 3.3 | 4.5 | 8.1 | 9.6 | 17.4 | 36 | 48 | 84 | 108 | 171 | 231 |

Section II Wiring

2.1 System construction and wiring

2.1.1 Servo driver wiring diagram

2.1.2 Wiring instructions

Wiring Notes:

- Use the wiring materials according to electric wire specification.

- The control cable length should be less than 3 meters and the encoder cable length 20 meters.

- Check that the power supply and wiring of R/L1、S/L2、T/L3 and R1、R2 terminals are correct. Please do not connect to 380V power supply.

- The output terminals(U/T1、V/T2、W/T3) must be connected with the servo motor connections correspondently, otherwise the servo motor will stop or over speed. However, by exchanging three-phase terminal cannot cause the motor to reverse; this is different from an asynchronous motor.

- Earthed wiring must be reliable with a single-point connection.

- Pay attention to the correct direction of freewheel diode which is connected with the relay at the output terminal, otherwise can cause the output circuit breakdown.

- In order to protect the servo driver from noise interference that can cause malfunction, please use an insulation transformer and noise filter on the power lines.

- Wiring the power lines (power supply line, main circuit lines, etc.) at a distance above 30cm from the control signal wires, do not lay them in one wiring tube.

- Install a non-fuse circuit breaker that can shut off the external power supply immediately in case of the servo driver fault.

2.1.3 Electric wire specifications

| Connect terminal | symbol | Wire specification |
| --- | --- | --- |
| Main power supply | R/L1、S/L2、T/L3 | 1.5～2.5mm2 |
| Control power supply | R1、R2 | 0.75～1.0mm2 |
| Servomotor | U/T1、V/T2、W/T3 | 1.5～2.5mm2 |
| Ground |  | 1.5～2.5mm2 |
| Control signals | X1 | ≥0.14mm2(AWG26), shielded |
| Encoder signals | X2 | ≥0.14mm2(AWG26), shielded |

Must use a twisted pair wire cable for the encoder signal wiring. If the encoder signal cable is too long (>20m), in which the encoder power supply can be insufficient, may use multi-wire or thick wire for the power supply wiring.

2.1.4 Servo motor and AC power supply wiring diagrams

The power supply for the servo driver is a three-phase AC 220V which generally come from three-phase AC380V power supply through a transformer. In peculiar circumstance, the small servomotor, which is less than 1.2kW, can use single-phase AC220V (R/L1、S/L2 terminals connect to single-phase power supply. Leave T/L3 terminal alone).

2.1.5 Main circuit terminal explanation

| Terminal name | Symbol | Detailed explanation |
| --- | --- | --- |
| Main power supply | R/L1、S/L2、T/L3 | Connect to external AC power supply 3 phase 220VAC -15%～+10% 50/60Hz |
| Control power supply | R1、R2 | Connect to external AC power supply 1 phase 220VAC -15%～+10% 50/60Hz |
| Servomotor | U/T1 | U phase output to servomotor |
|  | V/T2 | V phase output to servomotor |
|  | W/T3 | W phase output to servomotor |
| Ground |  | Ground terminal of servomotor |
|  |  | Ground terminal of servo driver |

2.2 X1 terminals for control signals

The X1 connector DB26 plug provides the signals interfaced with the host-controller. The signal includes:

- Five programmable inputs;

- Three programmable outputs;

- Analog command inputs;

- Pulse command inputs;

- Encoder signal outputs.

2.2.1 X1 terminal connector

The X1 connector plug uses DB26 male head, the contour and pin disposition charts are as the followings:

2.2.2 X1 terminal signal instructions

| # Name of signals |  | Pin # number | Functions | # Connector |
| --- | --- | --- | --- | --- |
| # Digital inputs | # DI1 | # 5 | # Photo isolation input; # Programmable functions; # Defined by parameters P100 to P104. | # C1 |
|  | # DI2 | # 14 |  |  |
|  | # DI3 | # 4 |  |  |
|  | # DI4 | # 13 |  |  |
|  | # DI5 | # 3 |  |  |
|  | # DI7 | # 2 |  |  |
|  | # +24V | # 1 | # DI power supply (DC12V～24V) |  |
| # Digital outputs | # DO1 | # 7 | # Photo isolation output; # maximum output: 50mA/25V; # function programmable; # defined by parameter F130~F132. | # C2 |
|  | # DO2 | # 16 |  |  |
|  | # DO3 | # 6 |  |  |
|  | # DOCOM | # 10 | # DO common terminal |  |
| # position command pulse | # PULS+ | # 12 | # High speed photo isolation input; Working mode set by parameter F035: # pulse + mark; # positive/reverse pulse; # Orthogonal pulse. | # C3 |
|  | # PULS- | # 11 |  |  |
|  | # SIGN+ | # 8 |  |  |
|  | # SIGN- | # 9 |  |  |
| # Analog command inputs | # AS+ | # 17 | # Speed/torque analog input; # the range is -10V to + 10V. | # C4 |
|  | # AS- | # 18 |  |  |
|  | # AGND | # 26 | # Analog ground |  |
| # Output signals of encoder | # OA+ | # 19 | # Outputs of differential driver (Line Driver) after the frequency division of encoder signal. | # C5 |
|  | # OA- | # 20 |  |  |
|  | # OB+ | # 21 |  |  |
|  | # OB- | # 22 |  |  |
|  | # 0Z+ | # 24 |  |  |
|  | # OZ- | # 23 |  |  |
|  | # CZ | # 15 | # Open collector output of Z signal | # C6 |
|  | # GND | # 25 | # Encoder signal ground |  |
| # Shielded cable ground protection | # Metal case of # connector | # - | # Shielded wire for connection with shielded cable. | # - |
|  |  | # - |  |  |

2.2.3 X1 terminal interface type

The followings introduce the X1 various interface circuits and the wiring ways with the host-controller.

1. Digital input interfaces (C1)

For carrying on a control, the digital input interface circuit can be constructed by switch, relay, open-collector triode, and photo-coupler and so on. To avoid contacting problem the relay must be chosen with low current operation. External voltage is in the range of DC12V~24V.

C1-1：Switch input

C1-2：Open collector triode

2. Digital output interfaces (C2)

The digital outputs use Darlington photo-coupler. It can be connected with relay, photo-coupler. Matters of note are:

- Inverting the polarity of DC power source, which is provided by the user, can cause the servo driver damage.

- The maximum voltage of external DC power supply is 25V, the maximum output current is 50mA, and the total current for three channels is not in excess of 100mA.

- When using relay like inductive loads, a free-wheel diode must be connected with the inductive load in parallel. If the diode connects in wrong direction can cause damage to the output circuit.

- Owing to the low level of output is approximately 1V and cannot satisfy the TTL low-level request, therefore cannot directly connect with the TTL circuit.

C2-1: Relay

★Freewheel diode must be connected.

C2-2: photo coupler

3. Position pulse command interfaces (C3)

There are both differential and single end connections. The differential connection is recommended and the twisted pair wire is used suitably. The drive current is in the range of 8 to 15mA.The operation mode is set by parameter F035: Pulse + direction, CCW/ CW pulse, A phase + B phase (orthogonal pulse).

C3-1: Differential drive

★Maximum pulse frequency is 500kHz(kpps)；

This connection is recommended in order to avoid interference.

C3-2: single end drive

★Maximum pulse frequency 200kHz(kpps)

Recommanded resistance R value：

| VCC | R |
| --- | --- |
| 5V | 82Ω~120Ω |
| 12V | 510Ω~820Ω |
| 24V | 1.5kΩ~2kΩ |

4. Line driver outputs of the encoder signals (C5)

The signal divided from the encoder signal is transferred to the host-controller through the line driver.

C5-1: Long line receiver

★On the host controller uses AM26LS32(or equivalent) to make the receiver, must connect the terminal resistance, the value is 220Ω～470Ω;

★Encoder signal (GND) of servo driver must connect with the ground terminal on host controller.

C5-2: photo coupler receiver

★On host controller use high-speed photo coupler (e.g. 6N137); Current limiting resistor is about 220Ω.

5. Open-collector output of encoder Z signal (C6)

The Z signal of the encoder is transferred to the host-controller through the open-collector circuit. Because the width of the Z pulse is narrow, please use a high-speed photo-coupler to receive it.

C6: Open collector output of encoder Z signal

★30V is the maximum voltage of external power supply; 50mA is the maximum current output.

2.2.4 Digital input definition

Every digital input interface is programmable, it can act different function by setting the corresponding parameter. The manufacturer sets it as the default value, users may modify it according to the need.

| Parameter | Name | Range | Default value | Unit | Usage |
| --- | --- | --- | --- | --- | --- |
| F100 | Function of digital input DI1 | -21～21 | 1 | - | ALL |
| F101 | Function of digital input DI2 | -21～21 | 2 | - | ALL |
| F102 | Function of digital input DI3 | -21～21 | 3 | - | ALL |
| F103 | Function of digital input DI4 | -21～21 | 4 | - | ALL |
| F104 | Function of digital input DI5 | -21～21 | 20 | - | ALL |

The absolute value of the parameter expresses functions; the symbolic expresses the logic, Positive number expresses positive logic and the negative number express the negative logic (ON is effective, OFF is invalid):

| Parameter | DI input signal | DI result |
| --- | --- | --- |
| Positive number | Turn off | OFF |
|  | Turn on | ON |
| Negative number | Turn off | ON |
|  | Turn on | OFF |

DI function table：

| Ordinal | Symbol | DI Function |
| --- | --- | --- |
| 0 | NULL | No function |
| 1 | SON | Servo enable |
| 2 | ARST | Clear alarm |
| 3 | CCWL | CCW drive inhibition |
| 4 | CWL | CW drive inhibition |
| 5 | TCCW | CCW torque limitation |
| 6 | TCW | CW torque limitation |
| 15 | EMG | Emergency stop |
| 18 | GEAR1 | Electronic gear switching 1 |
| 19 | GEAR2 | Electronic gear switching 2 |
| 20 | CLR | Clear position deviation |
| 21 | INH | Pulse input inhibition |

2.2.5 Digital output definition

Every digital output interface is programmable, it can act different function by setting the corresponding parameter. The manufacturer sets it as the default value, users may modify it according to the need.

| Parameter | Name | Range | Default value | Unit | Usage |
| --- | --- | --- | --- | --- | --- |
| F130 | Function of digital output DO1 | -11～11 | 2 | - | ALL |
| F131 | Function of digital output DO2 | -11～11 | 3 | - | ALL |
| F132 | Function of digital output DO3 | -11～11 | 8 | - | ALL |

The absolute value of the parameter expresses functions, the symbol expresses the logic. '0' is forcing OFF, '1' is forcing ON. The symbol indicates the output logic, the positive number expresses the positive logic and the negative number expresses the negative logic:

| Parameter value | DO output signal | DO result |
| --- | --- | --- |
| Positive number | Turn on | ON |
|  | Turn off | OFF |
| Negative number | Turn off | ON |
|  | Turn on | OFF |

DO function table：

| Ordinal | Symbol | DO Function |
| --- | --- | --- |
| 0 | OFF | Always invalid |
| 1 | ON | Always valid |
| 2 | RDY | Servo ready |
| 3 | ALM | Alarm |
| 5 | COIN | Positioning complete |
| 6 | ASP | Arrival speed |
| 8 | BRK | Electromagnetic brake |
| 11 | TRQL | Torque under limitation |

2.3 X2 encoder signal terminals

2.3.1 X2 terminal connector

# The encoder signal connector X2 connects with the servomotor encoder. A three row of DB15 plugs (the VGA plug) is used. The contour and pin disposition charts are:

2.3.1 X2 terminal signal description

| Signal name of encoder |  | Pin number | Colour of wire |  | Functions |
| --- | --- | --- | --- | --- | --- |
|  |  |  | standard (16core) | Wire saving (10core) |  |
| Power supply | V50 | 13 | red+red /white | red+red /white | Use 5VDC power supply (provided by servo driver). If the cable is longer than 20m, in order to prevent encoder from voltage drop down, it is better to use multi-wire or thick wire for power line and ground line. |
|  | GND | 14 | black+black /white | black+black /white |  |
| A phase input | A+ | 5 | brown | brown | Connect with A phase output of encoder. |
|  | A- | 10 | Brown/white | Brown/white |  |
| B phase input | B+ | 4 | yellow | yellow | Connect with B phase output of encoder. |
|  | B- | 9 | Yellow /white | Yellow /white |  |
| Z phase input | Z+ | 3 | green | green | Connect with Z phase output of encoder. |
|  | Z- | 8 | Green/white | Green/white |  |
| U phase input | U+ | 2 | purple | - | Connect with U phase output of encoder. Not connect for wire saving. |
|  | U- | 7 | Purple /white |  |  |
| V phase input | V+ | 1 | blue | - | Connect with V phase output of encoder. Not connect for wire saving. |
|  | V- | 6 | Blue/white |  |  |
| W phase input | W+ | 12 | orange | - | Connect with W phase output of encoder. Not connect for wire saving. |
|  | W- | 11 | Orange /white |  |  |
| Shield ground | FG | 15 | Bare wire | Bare wire | Connect with cable shield wire. |

Note: The optional extras provided:

1. 16 core cable for the type of 16S1P15.

2. 10 core cable for the type of 10S1P15X (for using in the 80 frame of servomotor) and of 10S1P15 (for using in the 110 and above frame of servomotor).

2.4 Standard wiring diagram

2.4.1 Wiring diagram for position control

Section III Front panel operation

3.1 Description of the front panel of servo driver

3.1.1 Panel compositions

The front panel consists of 5-digit LED display , four switching buttons 、、、 and two LED lights. It displays monitor status, parameters and changes the parameter setting value and so on. The main menu is in cascade sequence mode and executes in layer.

3.1.2 Panel description

| Symbol | Name | Functions |
| --- | --- | --- |
| POW | Main power light | ON: Main power supply already turned on; OFF: Main power supply not turned on. |
| RUN | Running light | ON: Servomotor is active; OFF: Servomotor is not active. |
|  | Increasing button | Increase sequence number or value; Press down and hold to repeat increasing. |
|  | Decreasing button | Decrease sequence number or value; Press down and hold to repeat decreasing. |
|  | Exit button | Menu exit; cancel the operation. |
|  | Confirm button | Menu entered; confirm the operation. |

3.1.3 Data display

A number is shown by 5-digit display; a minus symbol in front of the value represents a negative value; the lit decimal points in all the digits indicate a 5-digit negative value. Some displays have a prefix character. If the value is full-scale, then the prefix character can be omitted.

3.2 Main menu

The first layer is the main menu and has four operating modes. Press  、 button to change the operation mode. Press the  button to enter the second layer and then execute a specific operation. Press  button to return to the main menu from the second layer.

3.3 Status monitor

Choose status monitor "" under the main menu. Press the  button to enter the monitor mode. There are many kinds of monitor's projects; Use  and  button to select the needed project. Press the  button again to enter the specific display status.

1. 32 binary bits value display [note1]

32 binary bits value translates into a decimal value that is in the range of -2147483648~147483647. It is divided into the low portion and the top portion. Select the needed portion through the menu. By the following formula, the complete value can be obtained.

2. Pulse unit [note2]

The original position command pulse is the input pulse count that has not been transformed through the electronic gear. The pulse count unit for other parts is the same with the encoder pulse unit. Take 2500-line encoder as an example.

Encoder pulse unit = encoder resolution

= 4 × encoder line= 4 × 2500( pulse / rev)= 10000( pulse / rev)

3. Motor current [note3]

The servomotor current is Irms.

4. Peak torque and peak current [note4]

The maximum torque and maximum Irms of the servomotor in previous 10-second duration is defined as the peak value.

5. Position command pulse frequency [note5]

The frequency of position command pulse is the actual pulse frequency before the electronic gear. The positive number is shown as positive direction and the negative number as reverse direction.

6. Input terminals DI [note6]

A vertical segment of LED shows an input status. The lit top vertical segment shows the DI input to be “ON” and the lit bottom vertical segment to be “OFF”.

7. Output terminals DO [note7]

A vertical segment of LED shows an output status. The lit top vertical segment shows the DO output to be “ON” and the lit bottom vertical segment to be “OFF”.

8. Input signals from encoder [note8]

A vertical segment of LED shows an input status. The lit top vertical segment shows a HIGH-level signal and the lit bottom vertical segment a LOW-level signal.

9. Absolute position of rotor [note9]

The rotor position is relative to the stator in one revolution per cycle. Use the encoder pulse unit and take the encoder Z pulse as the zero point. Take 2500-line encoder as an example. The position of the rotor is in the range of 0~9999 and is zero when Z pulse appears.

10. Control mode [note10]

The first three characters show the control mode, the final character shows gain group.

11. Alarm code [note11]

The "Err" followed by two minus symbols indicates no alarm and by digital number indicates an error code number that is flickering. When alarm appears, the error code number displays automatically on the front panel LED. During the error status, the monitor mode can be changed to other modes by pressing buttons, but the decimal point of the last LED is still flickering and shows existence of an alarm.

3.4 Parameter settings

Parameters are displayed through the combination of parameter segment & parameter number, 100 bit represents segment number ,10 bit and 1 bit represent parameter number.Take F102 parameter as an example, '1' is the section name and '02' the parameter name. Front panel displays “”.

Choose the parameter mode “” under the main menu，press  button to enter parameter setting mode. First use  or  button to select the parameter section number and then press  button to enter the parameter number selection. Again, use  or  button to select the parameter number and then press  button to display the parameter value.

Use  or  button to modify a parameter value. By pressing  or  button once to increase or decrease the parameter value by one. Press down and hold the  or  button, the parameter value can increase (decrease) continuously. When the parameter value is modified, the decimal point on the most right side LED is lit. Press  button to confirm the parameter value to be effective, meanwhile the decimal point turns off. The modified parameter value is immediately active to influence on the control action (some parameters can only be valid when they have been stored before re-up electricity). Hereafter press  button to return to the parameter number selection and can continue to modify a parameter. If the value is not satisfied, do not press the  button but press  button to cancel it for resuming the original parameter value.

The modified parameter would not be stored in EEPROM. For permanent preservation, please refer to the parameter writing operation in the parameter management (3.5 sections). The parameter section name and the parameter number are not necessarily continual, but the parameter section name and the parameter number that are not in use will be jumped over and cannot be chosen.

3.5 Parameter management

Choose the parameter management mode under the main menu "". Press the  button to enter the parameter management mode. The operation is performed between parameter list and the EEPROM.

There are three operation modes. Use 、 buttons to select an operation mode and then press down and hold the  button at least three seconds to active the operation mode. After finished the operation and then press  button to return to the operation mode selection state.

- Write and save parameters

This operation indicates that the parameter in parameter list will write to the EEPROM. When user has made change to a parameter, it only change the parameter value in parameter list, but for the next time when the power supply is on the parameter value will restore its original value. Making permanent change to a parameter value, it is the need to carry out the parameter write operation and write the parameter value to the EEPROM. Hereafter, when the power supply is on again will be able to use the new parameter value.

- Read and obtain parameters

This operation indicates that all the parameters will be read from the EEPROM to the parameter list. This process will carry out automatically one time when power supply is on. At the beginning, the value of each parameter in the parameter list is the same as the parameter in the EEPROM. After making change to a parameter value, the value in the parameter list will also change. When the parameter value is not satisfied or comes to confusion, carries out the parameter read operation to read back the original parameter value from the EEPROM to the parameter list.

- Restore to default value

This operation indicates that each default value of all the parameters will read from EEPROM and write to the parameter list and EEPROM. For the next time when power supply is on the default parameters will be used by now. When many parameters become confusion and cause abnormal operation, it is necessary to carry out this operation for resuming the default parameters. There are different default parameters for different servo driver model and the servomotor model. Therefore, before doing this operation the servo driver code (Parameter F001) and the servomotor code (Parameter F002) must be set correctly.

3.6 Auxiliary functions

Choose the auxiliary function mode "" under the main menu. Press the  button to enter the auxiliary function mode. Use  or  button to select an operation mode. Then press the  button again to enter the corresponding function. After finished this operation pressing the  button to return to the operation mode selection state.

3.6.1 Special functions

Use for the manufacturer.

3.6.2 Jog function

Choose the JOG running "" of the auxiliary function. Press the  button to enter the JOG running mode. The "" symbol is as a prompt of spot movement. The numerical value is the speed command provided by F076 parameter and the unit is r/min. Press down and hold the  button, the servomotor will rotate in counterclockwise direction with JOG speed. Loose the button, the servomotor stops rotation and keeps zero speed. Alternatively, press down and hold the  button, the servomotor will rotate in clockwise direction with JOG speed.

3.6.3 Speed adjustment by keyboards

Choose keyboard speed adjustable “”, press  button to enter in keyboard speed adjustable mode. The "" symbol is as a prompt of keyboard speed adjustment, the unit of the value is r/min, speed command is offered by the buttons. Speed command could be adjusted through 、 buttons, and the motor would run according to the reference speed. Positive number represents forward running (CCW), negative number represents reverse running (CW), the minimum reference speed is 0.1r/min.

3.6.4 Zero set for analog quantity

No function for this device.

3.7 Restore to the parameter default values

In case of the following situation, please use the function of restoring the default parameter (manufacture parameter):

1）The parameter is adjusted chaotically, the system is unable to work normally.

2）The servomotor is replaced by a different newly model.

3）For other reasons, the servo driver code (parameter F001) does not match with the servomotor code (parameter F002).

The procedures for restoring the default parameter values are as the followings:

1) Inspect whether servo driver code (parameter F001) is correct or not.

2) Inspect whether servomotor code (parameter F002) is correct or not. If it is not correct, carry out following step, or, jump to step 5.

3) Modify the password (parameter F000) by 369.

4) Modify the servomotor code (parameter F002) with the needed servomotor code, refer to chapter 7.4 for servomotor adaptive table.

5) Enter the parameter management and carry out one of following operations:

a. Restore part of the parameter default value

For restoring default parameters related to the servo driver and the servomotor and maintaining the other user parameters, carry out the parameter write operation in the parameter management. This operation is active only when the password is 369 and the servomotor code has been modified. In other situations, it only has the parameter write function.

b. Restore all the parameter default value

Carry out to restore the default value in the parameter management, all the parameters including the parameters which have been modified by the user restore to the default value.

6) Turn off and on the power supply, then an operation can be performed again.

Section IV Running

4.1 Trial running with no load

The purpose of trial running is to confirm if the following items are correct or not:

- The wiring of servo driver power supply;

2)	The wiring of servomotor;

3)	The wiring of encoder;

4)	The running direction and speed of the servomotor.

4.1.1 Wiring and inspection

Before turn on the power supply, confirm the servomotor of:

- The servomotor has no load on the shaft; decoupling from the machinery if already coupled.

2)	Because the servomotor has an impact during acceleration or deceleration, the servomotor must be fixed.

Follow the wiring chart, inspect the following items before turning on the power supply:

1)	The wirings are correct or not. In particular, R/L1、S/L2、T/L3 wirings and U, V, W wirings corresponded to the servomotor are correct or not.

2)	The input voltage is correct or not.

3)	The encoder cable connection is correct or not.

4.1.2 Trial running in JOG mode

1. Power on

Turn on the control power supply (while the main power supply temporarily turned off). The front panel display is lit. If any error appears, please inspect the wirings. Then turn on the main power supply, the POWER indicating LED is lit.

2. Parameter settings

Set parameters according to the following table:

| Parameter | Name | Set value | Default value | Parameter description |
| --- | --- | --- | --- | --- |
| F004 | Control mode | 1 | 0 | Set speed control |
| F025 | Source of speed command | 3 | 3 | Set JOG source |
| F060 | Acceleration time of speed command | suitable | 0 | Decrease acceleration impact |
| F061 | Deceleration time of speed command | suitable | 0 | Decrease deceleration impact |
| F076 | JOG running speed | 100 | 100 | JOG speed |
| F097 | Neglect inhibition of servo driver | 3 | 3 | Neglect CCW inhibition (CCWL) and CW inhibition (CWL). |
| F098 | Forced enable | 1 or 0 | 0 | Set 1 for forced enable; Set 0 for external enable. |
| F100 | Digital input DI1 function | 1 | 1 | Set DI1 for servo enable (SON) |

3. Operation

Confirm that there is no alarm and any unusual situation, turn on the servo enable (SON), the RUN indicating LED lit and the servomotor is active at zero speed.

Choose the JOG running "" in the auxiliary function. Press the  button to enter the JOG running mode. The numerical value is the speed command provided by F076 parameter and the unit is r/min. Press down and hold the  button, the servomotor will rotate in counterclockwise direction with the JOG speed. Loosen the pressed button, the servomotor stops and keeps zero speed. Alternatively, press down and hold the  button, the servomotor will rotate in clockwise direction with the JOG speed.

4.1.3 Trial running in speed adjustment mode with keyboard

1. Power on

Turn on the control power supply (while the main power supply temporarily turned off). The front panel display is lit. If any error appears, please inspect the wirings. Then turn on the main power supply, the POWER indicating LED is lit.

2. Parameter setting

Set parameters according to the following table:

| Para- meter | Name | Set value | Default value | Parameter description |
| --- | --- | --- | --- | --- |
| F004 | Control mode | 1 | 0 | Set speed control |
| F025 | Source of speed command | 4 | 3 | Set JOG source |
| F097 | Neglect inhibition of servo driver | 3 | 3 | Neglect CCW inhibition (CCWL) and CW inhibition (CWL). |
| F098 | Forced enable | 1 or 0 | 0 | Set ‘1’for forced enable; Set ‘0’for external enable. |
| F100 | Digital input DI1 function | 1 | 1 | Set DI1 for servo enable (SON) |

3. Operation

Confirm that there is no alarm and unusual situation, turn on the servo enable (SON), the RUN indicating LED lit and the servomotor is active at zero speed.

Choose the adjustable speed "" in the auxiliary function. Press the  button to enter the adjustable running mode. The numerical value is the speed command provided by pressing  button or  button and the unit is 0.1r/min. Follow the speed command, the servomotor is in rotation. The rotation direction is dependent on the sign of digits. The positive number indicates forward direction (CCW), the negative number indicates reverse direction (CW).

4.1.4 Operation of analog control

Analog control and position control can not be valid simultaneously.

1. Example for analog control mode

This is a simple example of analog control. The wiring diagram is shown as below.

The parameter setting for the example:

| Para-meter | Name | Set value | Default value | Parameter description |
| --- | --- | --- | --- | --- |
| F004 | Control mode | 0 | 0 | - |
| F097 | Neglect inhibition of servo driver | 0 | 3 | Use CCW inhibition (CCWL) and CW inhibition (CWL). If neglect,may not connect CCWL、CWL. |
| F100 | Digital input DI1 function | 1 | 1 | Set DI1 as servo enable SON |
| F101 | Digital input DI2 function | 2 | 18 | Set DI2 as electric gear selection 1 |
| F102 | Digital input DI3 function | 3 | 19 | Set DI3 as electric gear selection 2 |
| F130 | Digital output DO1 function | 1 | 1 | Set DO1 as servo ready RDY |
| F029 | 1st numerator of electronic gear | 1 | 40 | - |
| F030 | Denominator numerator of electronic gear | 1 | 1 | - |
| F035 | Input mode of command pulse | 0 | 0 | - |
| F036 | Phase of input command pulse | 0 | 0 | - |
| F037 | Signal logic of input command pulse | 0 | 0 | - |
| F038 | Signal filter of input command pulse | 7 | 7 | - |
| F039 | Filter mode of input command pulse | 0 | 0 | - |
| F040 | Time-constant of exponential form filter for position command | 0 | 40 | - |

4.2 Position control mode

The position control is applied in systems where precise location is necessary, such as numerical control machine tool, textile machinery and so on. The position command is a pulse serial coming from the input terminals PULS+, PULS-, SIGN+ and SIGN- .

4.2.1 Example for position control mode

This is a simple example of position control. The wiring diagram is shown as below.

The parameter setting for the example:：

| Para-meter | Name | Set value | Default value | Parameter description |
| --- | --- | --- | --- | --- |
| F004 | Control mode | 0 | 0 | Set as position control |
| F097 | Neglect inhibition of servo driver | 0 | 3 | Use CCW inhibition (CCWL) and CW inhibition (CWL). If neglect, may not connect CCWL、CWL. |
| F100 | Digital input DI1 function | 1 | 1 | Set DI1 as servo enable SON |
| F130 | Digital output DO1 function | 1 | 1 | Set DO1 as servo ready RDY |

4.2.2 Position command

1. Related parameters of position command

| Para-meter | Name | Range | Default value | Unit | Usage |
| --- | --- | --- | --- | --- | --- |
| F029 | 1st numerator of electronic gear | 1～32767 | 1 | - | P |
| F030 | Denominator of electronic gear | 1～32767 | 1 | - | P |
| F031 | 2ed numerator of electronic gear | 1～32767 | 1 | - | P |
| F032 | 3rd numerator of electronic gear | 1～32767 | 1 | - | P |
| F033 | 4th numerator of electronic gear | 1～32767 | 1 | - | P |
| F035 | Input mode | 0～2 | 0 | - | P |
| F036 | Input direction | 0～1 | 0 | - | P |
| F037 | Signal logic of input command pulse | 0～3 | 0 | - | P |
| F038 | Signal filter of input command pulse | 0～21 | 7 | - | P |
| F039 | Filter mode of input command pulse | 0～1 | 0 | - | P |
| F040 | Time-constant of exponential form filter for position command | 0～1000 | 0 | ms | P |

2. Transmission path of command pulse

3. Input mode of command pulse

The command pulse input mode is dependent on the parameter F035. For adjusting the counting edge of a pulse, the parameter F037 sets the phases of the PULS and the SIGN signals. Parameter F036 is used in changing the counting direction.

Forms of pulse command:

- Pulse+direction (Set F035 to 0)

- Forward/reverse pulse (Set F035 to 1)

- Orthogonal pulse (Set F035 to 2)

Note: The arrow indicates the counting edge with F306=0 and F307=0.

4. Timing sequence specification of pulse command

Pulse wave form of pulse command：

- Pulse+direction

- Forward/reverse pulse

- Orthogonal pulse

Remark：Parameter requirement

| Differential | tck>2us,th>1us,tl>1us,trh<0.2us,trl<0.2us,ts>1us,tqck>8us,tqh>4us,tql>4us,tqrh<0.2us,tqrl<0.2us,tqs>1us |
| --- | --- |
| Single end | tck>5us,th>>2.5us,tl>>2.5us,trh<0.3us,trl<0.3us,ts>2.5us,tqck>10us,tqh>5us,tql>5us,tqrh<0.3us,tqrl<0.3us,tqs>2.5us |

5. Signal filter

Numeral filters related to the parameter F038 will filter the input signal PULS and SIGN. The bigger the F038 value, the larger filter time-constant and the lower maximum repeated frequency of input pulse. The maximum pulse input frequency is 500kHz(kpps) under the default value. The bigger the value, the smaller the maximum pulse input frequency would be.

If the positioning is not accurate, increase the parameter F038 in order to filter noise on the signal cable and to avoid counting error. The SIGN filter can be close by parameter P039 setting.

6. Smooth filter

The parameter F040 carries on the smooth filter to the command frequency. It has the exponential form of acceleration and deceleration as shown in the following chart. The filter cannot lose any input pulse, but can delay its action time. When F040 is zero, the filter does not have any effect. The parameter value indicates the time in which the repeated frequency increases from 0 to 63.2% command frequency.

The filter makes the input repeated frequency smooth. This filter is used in the following situations: the host controller without acceleration and deceleration function; the electronic gear ratio is quite big; the command frequency is relatively low.

4.2.3 Electronic gear for input commands

Through the electronic gear user can define that one input command pulse will cause an adjustable movement of mechanical device. Therefore, the host controller does not have to consider that the gear ratio in the mechanical system and the encoder line number of the servomotor .The electronic gear variables are illustrated in the following table.

| Variable | Description | Value of the set |
| --- | --- | --- |
| C | Lines of encoder | 2500 |
| Pt | Resolution of encoder (pulse/rev) | =4xC =4x2500 =10000（pulse/rev） |
| R | Ratio of deceleration | R=B/A, among which A:turn number of servomotor； B:turn number of load shaft |
| △P | One command pulse travel equivalent | - |
| PC | command pulse numbers for one turn of the load shaft | - |
| Pitch | command pulse numbers for one turn of ball bearing shaft (mm) | - |
| D | Diameter of rolling cylinder (mm) | - |

Calculating formula：

Electronic gear ratioNM=Resolution in one turn of encoderPtCommand pulse number in one turn of load shaftPc× reducer ratioR

Here,

Command pulse number in one turn of load shaft Pc          =Movement quantity in one turn of load shaftMovement quantity in one command pulse∆P

The calculated result will be abbreviated and make the numerator and the denominator smaller or equal to 32767 integer values. At last, the result must be in the range of 1/50<N/M<200 and write to the parameter list.

1. Electronic gear is used for ball screw drive

For the load of ball-screw:

Electronic gear ratioNM=PtPc×R

Among which，

Pc=Pitch∆P

For example:

Given that, encoder line number C=2500 line, the deceleration ratio 1/1, pitches Pitch=8mm, a pulse travel equivalent ΔP=0.001mm. Calculate the electronic gear ratio.

Calculation steps:

Calculate the resolution of the encoder (Pt)

Pt=4×C=4×2500=10000（pulserev）

Calculate the command pulse numbers for one turn of the load shaft (ball-screw)（Pc）

Pc=Pitch∆P=8mm0.001mm=8000

Calculate the electronic gear ratio

Electronic gear ratioNM=PtPc×R=100008000×(11)=54

Set parameters (Take the first numerator as an example)

Numerator N=5，denominator M=4，set F029=5 and F030=4.

2. Electronic gear used for graduator drive

For graduator load, there is

Electronic gear ratioNM=PtPc×R

Among which，

Pc=360°∆P

For example:

Given that, encoder line number C=2500 line, the deceleration ratio 1/3, a pulse travel equivalent ΔP==0.1°. Calculate the electronic gear ratio.

Calculation steps:

Calculate the resolution of the encoder (Pt)

Pt=4×C=4×2500=10000(pulserev)

Calculate the command pulse numbers for one turn of the load shaft (ball-screw)（Pc）

Pc=360°∆P=360°0.1°=3600

Calculate the electronic gear ratio

Electronic gear ratioNM=PtPC×R=100003600×(13)=300003600=253

Set parameters (Take the first numerator as an example)

Numerator N=25，denominator M=3，set F029=25 and F030=3.

3. Electronic gear used for conveyer belt drive

For conveyer belt load, there is:

Electronic gear ratioNM=PtPc×R

Among which，

Pc=πD∆P

For example:

Given that, encoder line number C=2500 line, the deceleration ratio 1/10, idler wheel diameter D=200mm, a pulse travel equivalent ΔP=0.01mm. Calculate the electronic gear ratio.

Calculation steps:

Calculate the resolution of the encoder (Pt)

Pt=4×C=4×2500=10000(pulserev)

Calculate the command pulse numbers for one turn of the load shaft (ball-screw)（Pc）

Pc=πD∆P=3.14×2000.01=62800

Calculate the electronic gear ratio

Electronic gear ratioNM=PtPc×R=1000062800×（110）=10000062800=2500157

Set parameters (Take the first numerator as an example)

Numerator N=2500，denominator M=157，set F029=2500 and F030=157.

4. The relation between the electronic gear ratio and the turn number of servomotor

The relation between the electronic gear ratio and the turn number of servomotor is:

Servomotor turn number=pulse×NPt×M

Among which, pulse is input pulse number. For example, the encoder line number C=2500 line, N=20, M=3, pulse=1000, the calculation is:

Servomotor turn number=1000×2010000×3=23(circle)

5. The relation between the electronic gear ratio and the rotary speed of servomotor

Servomotor speedrmin=f(Hz)×60×NPt×M

Among which, f is the repeated frequency of the input pulse; unit is Hz (pps). For example, the encoder line number C=2500 line, N=3, M=1, f=100kHz (kpps), the calculation is:

Servomotor speedrmin=100×103×60×310000×1=1800（rmin）

6. Electronic gear ratio switching

Four groups of electronic gear numerator N are provided by the servo driver. The group can be changed online by signal of GEAR1 and GEAR2 from DI inputs. However, the denominator M is all the same.

| DI signal[note] |  | Numerator of input electronic gear N | Denominator of input electronic gear M |
| --- | --- | --- | --- |
| GEAR2 | GERA1 |  |  |
| 0 | 0 | 1st numerator(parameterF029) | Denominator (parameter F030) |
| 0 | 1 | 2nd numerator(parameterF031) |  |
| 1 | 0 | 3rd numerator(parameterF032) |  |
| 1 | 1 | 4th numerator(parameterF033) |  |

Note: 0 indicates OFF; 1 indicates ON.

4.2.4 Gains related to position control mode

| Param eter | Name | Range | Default value | Unit | Usage |
| --- | --- | --- | --- | --- | --- |
| F009 | Gain of position loop | 1～1000 | 40 | 1/s | P |
| F021 | Feed forward gain of position loop | 0～100 | 0 | % | P |
| F022 | Time-constant of feed forward filter for position loop | 0.20～50.00 | 1.00 | ms | P |

According to the inner loop adjusts first and then the outer loop, the speed loop is included in the position loop, therefore the rotation inertia ratio of load will be set first with suitable value. Then, the gain and the integral time-constant of the speed loop are adjusted. At last, the gain of the position loop is adjusted.

The following block diagram is the position regulator of the system. Increasing the gain of position loop Kp can get higher position loop bandwidth, but it is limited by the speed loop bandwidth. Therefore, in order to increase the gain of the position loop must increase the bandwidth of speed loop first.

The feed forward can reduce the lagging of phase in the position loop; and reduce the position tracking error as well as shorter positioning time. The feed forward quantity increases, the position tracking error reduces, but can cause the system unstable and overshoot if the feed forward quantity is too large. If the electronic gear ratio is more than 10 it is also easy to make noise. For normal application, the parameter F021 is set as 0%. If higher response and lower tracking error are required, the F021 can be increased properly, but not in excess of 80%. Meanwhile it may need to adjust the filter time constant (parameter F022) of the feed forward branch.

4.3 Gain adjustment

The servo driver includes the current control loop, the speed control loop and the position control loop. The control diagram is as follows:

Theoretically, the inner control loop bandwidth must be higher than the outer loop; otherwise, the entire control system will be unstable and creates the vibration or worse response. Therefore, the relations of the bandwidth of the three control loops are as follows:

Bandwidth of the current loop>bandwidth of the speed loop>bandwidth of the position loop

Because the current control loop of the servo driver is already adjusted in an optimum condition, users should only adjust parameters of speed control loop and position control loop.

4.3.1 Gain parameters

The parameters related to the gain:

| Param eter | Name | Range | Default value | Unit | Usage |
| --- | --- | --- | --- | --- | --- |
| F005 | Gain of speed loop | 1～3000 | 40 | Hz | P,S |
| F006 | Integral time constant of speed loop | 1.0～1000.0 | 20.0 | ms | P,S |
| F009 | Gain of position loop | 1～1000 | 40 | 1/s | P |
| F017 | Ratio of load inertia | 0.0～200.0 | 1.5 | times | P,S |

The definition of symbols are as follows:

KV：The gain of speed loop;

Ti：The integral time-constant of speed loop;

Kp：The gain of position loop;

G： The inertia ratio of load (F017);

JL：The load inertia referred to the rotor shaft;

JM：The rotor inertia of the servomotor.

1. The gain of speed loop Kv

The speed loop gain Kv directly determines the response bandwidth of the speed loop. Under the premise that there is no vibration in the mechanical system or noise, increases the speed loop gain, then the speed response can speed up, and is better to follow the speed command. However, it is easy to cause a mechanical resonance if t Kv is too large. The bandwidth of speed loop expresses as:

Speed loop bandwidthHz=1+G1+JLJM×KVHZ

If the setting inertia ratio of the load G is correct (G=JL/JM), then the bandwidth of the speed loop is equal to the speed loop gain Kv.

2. The integral time-constant of speed loop Ti

The integral item of speed loop can effectively eliminate static error of speed, and has rapid reaction to a slight speed change. Under the premise that there is no vibration in the mechanical system or noise, reduce the integral time constant Ti of speed loop, then the stiffness of the system increases, and reduce the static error. If load inertia ratio is very big or a resonating factor exists in the mechanical system, and then must confirm that the integral time constant is big enough, otherwise the mechanical system will be easy to cause resonating. If the setting inertia ratio of the load G is correct (G=JL/JM), use the following formula to obtain the integral time constant Ti of the speed loop.

Tims≥40002π×kv(HZ)

3. The gain of position loop Kp

The gain of the position loop directly determines the reaction rate of the position loop. Under the premise that there is no vibration in the mechanical system or noise, increase the position loop gain, then speed up the reaction rate, reduce the position tracking error and the positioning time is shorter. However, it is easy to cause a mechanical vibration or positioning overshoot if the Kp is set too large. The bandwidth of the position loop should be lower than the bandwidth of speed loop. In general:

Position loop bandwidth（HZ）≤Speed loop bandwidth（HZ）4

If the setting inertia ratio of the load G is correct (G=JL/JM), use the following formula to obtain the gain Kp of the position loop:

KP（1s）≤2π×KV（HZ）4

4.3.2. Procedure for gain adjustment

The bandwidth selections of the position and the speed loop depend on the machinery rigidity and the application situation. A leather belt conveyer has low rigidity and may set low bandwidth. Machinery with reducer and ball bearing screw has medium rigidity and may set medium bandwidth. Machinery with ball bearing screw or linear motor has higher rigidity and may set high bandwidth. If mechanical characteristics are unknown, may gradually increase the bandwidth until resonating, and then decrease the gain.

In the servo system, if changes a parameter, then other parameters also need to readjust. Therefore, do not change a parameter far from its original value. About the steps for changing the servo parameter, please observe the following principle:

| Increase response | Decrease response, restrain vibration and overshoot |
| --- | --- |
| 1.Increase gain of speed loop KV 2.Decrease integral time constant of speed loop Ti 3. Increase gain of position loop Kp | 1. Decrease gain of position loop Kp 2.Increase integral time constant of speed loop Ti 3. Decrease gain of speed loop Kv |

Gain adjustment procedure for speed control loop

1. Set the load inertia ratio.

2. Set integral time constant of the speed loop with a relatively great value.

3. Under no vibration and unusual sound increase the gain of the speed loop, if vibration occurs then decrease the gain a bit.

4. Under no vibration and unusual sound, decrease the integral time constant of speed loop, if vibration occurs then increase the time constant a bit.

5. Because the mechanical system may have resonating factors and is unable to adjust for a bigger gain, then the desired response cannot be obtained. Now, adjust the filter time constant (parameter F007) of torque, and then carry on the above steps again to enhance responsiveness.

Gain adjustment procedure for position control loop

1. Set the load inertia ratio.

2. Set integral time constant of the speed loop with a relatively great value.

3. Under no vibration and unusual sound increase the gain of the speed loop, if vibration occurs then decrease the gain a bit.

4. Under no vibration and unusual sound, decrease the integral time constant of speed loop, if vibration occurs then increase the time constant a bit.

5. Increase the gain of position loop, if vibration occurs then decrease the gain a bit.

6. Because the mechanical system may have resonating factors and is unable to adjust for a bigger gain, then the desired response cannot be obtained. Now, adjust the filter time constant (parameter F007) of torque, and then carry on above steps again to enhance responsiveness.

7. If need shorter positioning time and smaller position tracking error, can adjust the feed forward of the position loop. Please refer to section 4.2.4 for details.

4.4 Resonance suppressions

When the mechanical system has the resonance effect, it is possibly created by higher rigidity of the servo system and quicker response. It may be improved if reduce the gain. The servo driver provides the low pass filter. Under unchanging the gain by using filters can achieve the effect of resonance suppression.

The parameters related to resonating suppression are shown as follows:

| Para meter | Name | Range | Default value | Unit | Usage |
| --- | --- | --- | --- | --- | --- |
| F007 | Time constant of filter for torque | 0.10～50.00 | 2.50 | ms | ALL |

The low pass filter is active by default. The parameter F007 is used to set the time constant of torque filter. The low pass filter has the very good weaken effect on high frequency and can suppress high frequency resonance and noise. For example, the machinery with ball bearing screw sometimes can have high frequency resonance if increasing the gain. By using low pass filter can get better effect, but the system response bandwidth and the phase allowance also reduced, the system may become unstable.

When the high frequency vibration caused by the servo driver, adjust the filter time-constant Tf of torque, possibly can eliminate the vibration. The smaller the value, the better control response achieves, but it is limited by mechanical condition. The bigger the value, the better the suppressing effect achieved on high frequency vibration, but the phase margin reduces and can cause the oscillation if the value is too big. If the load inertia ratio G is set correctly (G=JL/JM), must meet the following needs:

Tfms≥10002π×2×kv(HZ)

4.5 Over-travel protection

The security function of over-travel protection refers to the movement part of the machinery that exceeding the designed safe range of motion, the limit switch acts and forces the servomotor to stop. Schematic diagram for over travel protection are shown as follows:

Normal closed contact is suggested for the limit switch. It is closed in the safety range and it is open in over travel range. The limit switch on the right connects to CCW forbid terminal (CCWL) and the limit switch on the left connects to CW forbid terminal (CWL).

This security function of over-travel protection can be set for use or neglect by setting the parameter F097. The limit signal must be connected for the use, or do not need this signal in the case of neglect.

The default value of CCWL and CWL are all neglects. Modify parameter F097 if needs to use. Even under the over-travel condition, use the reverse command to exit the over-travel condition is still allowable.

| F097 | Motion inhibition inCW direction | Motion inhibition in CCW direction |
| --- | --- | --- |
| 0 | Use | Use |
| 1 | Use | Neglect |
| 2 | Neglect | Use |
| 3(Default) | Neglect | Neglect |

4.6 Torque limitations

For protecting the machinery from over-load, users could limit the output torque.

4.6.1 Parameters for torque limitations

| Para meter | Name |  | Range | Default value | Unit | Usage |
| --- | --- | --- | --- | --- | --- | --- |
| F065 |  | Internal torque limit in CCW direction | 0～300 | 300 | % | ALL |
| F066 |  | Internal torque limit in CW direction | -300～0 | -300 | % | ALL |
| F067 |  | External torque limit in CCW direction | 0～300 | 100 | % | ALL |
| F068 |  | External torque limit in CW direction | -300～0 | -100 | % | ALL |
| F069 |  | Torque limit in trial running | 0～300 | 100 | % | ALL |

4.7 Timing sequence of operation

4.7.1 Timing sequence upon power on

1. The control power supply R1, R2 are turned on before or at the same time when the main power supply R/L1、S/L2、T/L3 turn on. If only the control power supply turn on, the servo ready signal (RDY) is OFF.

2. After the main power supply turned on, at about 1.5 seconds later the servo ready signal is on (RDY), from now can accept the servo enable signal (SON). The servo driver examines that the SON is effective, and then the power circuit and the servomotor are active. The servomotor is in running status. If the SON is invalid or an alarm occurs, power circuit shut down and the servomotor is in free running state.

4.7.2 Alarm timing chart upon servo-ON

4.7.3 Action timing chart while servo-ON/OFF are executed during the servo motor in standstill condition

When the speed of the servomotor is lower than parameter F165, the action-timing chart is:

4.7.4 Action timing chart while servo-ON/OFF are executed during the servo motor in motion condition

When the speed of the servomotor is higher than parameter F165, the action-timing chart is:

4.8 Electromagnetic brake

The electromagnetic brake (holding brake, lost power brake) is used in locking the vertical or the tilting table of machine tool, which connected with the servomotor. When the power supply lost or SON OFF, prevent the worktable from fall and break. To realize this function, select and purchase the servomotor with electromagnetic brake is needed. The brake can only be used for holding the worktable and cannot be used for deceleration and or stopping machine movement.

4.8.1 Parameters of electromagnetic brake

The parameters related to the electromagnetic brake:

| Para meter | Name | Range | Default value | Unit | Usage |
| --- | --- | --- | --- | --- | --- |
| F165 | Speed check point for servomotor static condition | 0～1000 | 5 | r/min | ALL |
| F166 | Delay time for electromagnetic brake for servo- motor static condition | 0～2000 | 0 | ms | ALL |
| F167 | Waiting time for electromagnetic brake for servo- motor operation condition | 0～2000 | 500 | ms | ALL |
| F168 | Action speed for electromagnetic brake for servo- motor operation condition | 0～3000 | 100 | r/min | ALL |

4.8.2 The use of electromagnetic brake

The chart below is the brake wiring diagram, the brake release signal BRK of the servo driver connect to the relay coil, the contact of relay connect brake coil and DC supply. The brake power supply has enough capacity provided by the user. Surge absorber is suggested to be installed to suppress surge voltage caused by switching off the relay. The diode also can be used as the surge absorber, but must pay attention to the slight brake delay.

When the speed of the servomotor is smaller than parameter F165, if the SON becomes OFF, the servomotor will continue to be energized for holding the position, after the period set by parameter P166, then remove the power supply from the servomotor.

When the speed of the servomotor is bigger than parameter F165, if the SON becomes OFF, the excitation is removed from the servomotor, after delay period of time the brake becomes active. During the delay time, the servomotor decelerates from the high speed down to the low speed, and then the brake is active to avoid damaging the brake. The delay time is set by the parameter F167 or is the time that the speed of the servomotor decelerates to the speed set by parameter F168. The delay time will take the smaller value.

Section V Parameters

5.1 Parameter table

The usage item in the table indicates the suitable control mode. “P” stands for the position control; “S” stands for the speed control; “T” stands for the torque control; “All” stands for the position, speed, and torque control. The“*”indicates default value that may be different.

5.1.1 Parameters of section 0

| Para- meter | Name | Range |  | Default value | Unit | Usage |
| --- | --- | --- | --- | --- | --- | --- |
| F000 | Password | 0～9999 |  | 315 | - | ALL |
| Classifying parameter management can guarantee the parameters from wrong modification. Set F000 to 315 can examine, modify the parameters of the 0, 1, 2 sections. For other settings can only examine, but not modify parameters. Suitable password is needed for the specific operations. |  |  |  |  |  |  |
| F001 | Identity code of servo driver | * |  | * | - | ALL |
| The current model of servo driver. It is manufacturer default value and cannot be modified by the users. L08：DXS1-0R8 L12：DXS1-1R2 L16：DXS1-1R6 L20：DXS1-2R0 |  |  |  |  |  |  |
| F002 | Identity code of servomotor | * |  | * | - | ALL |
| The current model of servomotor. It is manufacturer default value and cannot be modified by the users. For details please refer to chapter 7.4 of the adaptive table of servomotor. F002 should be modified when other types of motors being replaced, for details please refer to chapter 3.7 for details. |  |  |  |  |  |  |
| F003 | Software version | * |  | * | - | ALL |
| This is the software version number and cannot be modified. |  |  |  |  |  |  |
| F004 | Control mode | Position control | 0 | 0 | - | ALL |
|  |  | Speed control | 1 |  |  |  |
| 0： Position control mode 1： Speed control mode |  |  |  |  |  |  |
| F005 | Gain of speed loop | 1～3000 |  | 40 | Hz | P,S |
| This is the proportion gain of the speed regulator. Increase the parameter value, can speed up the speed response. It is easy to cause the vibration and the noise when the value is too large. If the F017 (load inertia ratio) is a correct value then the parameter value is equal to the speed response bandwidth. |  |  |  |  |  |  |
| F006 | Integral time constant of speed loop | 1.0～1000.0 |  | 20.0 | ms | P,S |
| This is the integral time constant of the speed regulator. Reduce the parameter value, can reduce the speed control error, and increase rigidity. It is easy to cause the vibration and the noise when the value is too small. Set maximum value (1000.0) as the cancel of integral function. The speed regulator becomes the P controller. |  |  |  |  |  |  |
| F007 | Filter time constant of torque | 0.10～50.00 |  | 2.50 | ms | ALL |
| This is the low pass filter of torque and can suppress the vibration of the machinery. The bigger the value, the better the effect of suppression. But excessive value may lead to the slow response or oscillation. The smaller the value, the quicker the response achieves, but can be limited by mechanical condition. When the load inertia is small, users may set a relative small value; the load inertia is big, users may set a relatively big value. |  |  |  |  |  |  |
| F009 | Gain of position loop | 1～1000 |  | 40 | 1/s | P |
| This is the proportional gain of the position regulator. Increases the parameter value, can reduce the position tracking error, and enhance the response. It is easy to cause overshoot or oscillation when the value is set too large. |  |  |  |  |  |  |
| F017 | Inertia ratio of load | 0.0～200.0 |  | 1.0 | 倍 | P,S |
| The load inertia ratio is the ratio of mechanical load inertia (refers to servomotor shaft) relatively to the rotor inertia of the servomotor. |  |  |  |  |  |  |
| F019 | Time constant of filter for speed detection | 0.50～50.00 |  | 2.50 | ms | P,S |
| The bigger value of parameter can get the smoother detected speed signal. The smaller value of parameter can get the quicker responded signal, but it will cause noise if the value is too small. In addition, it will cause oscillation if the value is too big. |  |  |  |  |  |  |
| F021 | Feed forward gain of position loop | 0～100 |  | 0 | % | P |
| The feed forward can reduce position-tracking error in the position control mode. Under any frequency command pulse the position-tracking error always becomes zero if the parameter set value is 100. Increasing the parameter value could enhance the response of position control. Excessive value may cause unstable state or oscillation of the system. |  |  |  |  |  |  |
| F022 | Time constant of feed forward filter for position loop | 0.20～50.00 |  | 1.00 | ms | P |
| For filtering the feed forward signal in position loop, the function is to increase the stability of feed forward control. |  |  |  |  |  |  |
| F025 | Sources of speed command | JOG speed c | 3 | 3 | - | S |
|  |  | Keyboard speed | 4 |  |  |  |
|  |  | Demonstration speed | 5 |  |  |  |
| Set the source of the speed command in speed control mode. The meanings of this parameter are: 3：This is the JOG speed command. It needs to set this parameter when begins using the JOG operation. 4：This is the keyboard speed command. It needs to set this parameter when begins using the (Sr) operation. 5：This is the demonstration speed command. It needs to set this parameter when begins using the demonstration operation. The speed command can change automatically. |  |  |  |  |  |  |
| F029 | First numerator of electronic gear for command pulse | 1～32767 |  | 1 | - | P |
| It is used for frequency division or multiplication of the input pulse and can conveniently match with each kind of the pulse source, in this way, to achieve the pulse resolution for the user’s needs. The electronic gear numerator N of command pulse is determined by GEAR1 and GEAR2 from DI inputs. The denominator M is set by parameter F030. DI Signals [note]; Numerator of electronic gear for command pulse N. GEAR2; GEAR1. 0; 0; The first numerator (parameter F029). 0; 1; The second numerator (parameter F031). 1; 0; The third numerator (parameter F032). 1; 1; The fourth numerator (parameter F033). Note: 0 indicates OFF; 1 indicates ON. The input pulse command becomes the position command by the N/M factor. The ratio range is: 1/50<N/M<200. |
| F030 | Denominator of electronic gear for command pulse | 1～32767 |  | 1 | - | P |
| This is electronic gear denominator M of command pulse. For method of application please refer to parameter F029 for details. |  |  |  |  |  |  |
| F031 | Second numerator of electronic gear for command pulse | 1～32767 |  | 1 | - | P |
| For details please refer to parameter F029. |  |  |  |  |  |  |
| F032 | Third numerator of electronic gear for command pulse | 1～32767 |  | 1 | - | P |
| For details please refer to parameter F029. |  |  |  |  |  |  |
| F033 | Fourth numerator of electronic gear for command pulse | 1～32767 |  | 1 | - | P |
| For details please refer to parameter F029. |  |  |  |  |  |  |
| F035 | Input mode of command pulse | Pulse + direction | 0 | 0 | - | P |
|  |  | Positive/reverse pulse | 1 |  |  |  |
|  |  | Orthogonal pulse | 2 |  |  |  |
| Set the input mode of command pulse: 0： Pulse + direction（Set parameter F035 to 0） 1： Positive/reverse pulse（Set parameter F035 to 1） 2： Orthogonal pulse（Set parameter F035 to 2） Note: The arrow indicates the counting edge when F036=0, F037=0. The diagram of command pulse inputs： Parameters should be preserved after modification, and become valid again after reset the power supply. |  |  |  |  |  |  |
| F036 | Input direction of command pulse | Normal direction | 0 | 0 | - | P |
|  |  | Reverse direction | 1 |  |  |  |
| 0： Normal direction 1： Reverse direction |  |  |  |  |  |  |
| F037 | Input signal logic of command pulse | 0～3 |  | 0 | - | P |
| Set the phase of the input pulse signals PULS and SIGN for adjusting the counting edge as well as the counting direction. F037; PULS signal; SIGN signal. 0; In phase; In phase. 1; Opposite phase; In phase. 2; In phase; Opposite phase. 3; Opposite phase; Opposite phase. Parameters should be preserved after modification, and become valid again after reset the power supply. |
| F038 | Input signal filter of command pulse | 0～21 |  | 7 | - | P |
| Filter the input signal PULS and SIGN numerically. The bigger the value, the larger the filter time constant. The maximum input pulse frequency is 500 kHz (kpps) when the set value is seven. If the value is bigger, the maximum input pulse frequency will reduce correspondingly. Filter the noise from the input signal to avoid counting mistake. If running is not perfect caused by the counting pulse, then can suitably increase the parameter value. Parameters should be preserved after modification, and become valid again after reset the power supply. |  |  |  |  |  |  |
| F039 | Input filter mode of command pulse | 0～1 |  | 0 | - | P |
| The meanings of this parameter are: 0：Filter the input signal PULS and SIGN numerically. 1：Filter the input signal PULS only and not filter the SIGN signal. Parameters should be preserved after modification, and become valid again after reset the power supply. |  |  |  |  |  |  |
| F040 | Time-constant of exponential form filter for position command | 0～1000 |  | 0 | ms | P |
| To conduct smoothing filtering for the command pulse with exponential form of acceleration/deceleration. The filter would not lose the input pulse, but can delay the command pulse. When the set value is zero, the filter does not have any effect. The filter is applied for: 1. The host controller has no acceleration/deceleration function; 2. The electronic gear ratio is quite big (N/M>10); 3. The command frequency is relatively low; 4. Stepping jump or unstable phenomenon appears during motor operation. |  |  |  |  |  |  |
| F060 | Acceleration time of speed command | 0～30000 |  | 0 | ms | S |
| Set the acceleration time for the servomotor from the zero speed up to the rated speed. If the command speed is lower than the rated speed, the acceleration time also correspondingly reduces. Only applied to the speed control mode. It is invalid in position control mode. If the position control is consisted of servo driver and position controller, the parameter should be set to 0, or the performance of the position control would be influced. |  |  |  |  |  |  |
| F061 | Deceleration time of speed command | 0～30000 |  | 0 | ms | S |
| Set the deceleration time for the servomotor from the rated speed down to zero speed. If the command speed is lower than the rated speed, the deceleration time also correspondingly reduces. Only applied to the speed control mode. It is invalid in position control mode. If the servo driver is applied with external positon loop, the parameter should be set to zero, or the performance of the position control would be influced. |  |  |  |  |  |  |
| F065 | Internal torque limit in CCW direction | 0～300 |  | 300 | % | ALL |
| Set the internal torque limitation value in CCW direction of servomotor. This limit is effective all the time. If the value exceeds the maximum overload capacity of the servo driver, then the actual limits will be equal to the system maximum overload capacity. |  |  |  |  |  |  |
| F066 | Internal torque limit in CW direction | -300～0 |  | -300 | % | ALL |
| Set the internal torque limitation value in CW direction of servomotor. This limit is effective all the time. If the value exceeds the maximum overload capacity of the servo driver, then the actual limits will be equal to the system maximum overload capacity. |  |  |  |  |  |  |
| F067 | External torque limit in CCW direction | 0～300 |  | 100 | % | ALL |
| Set the external torque limitation value in CCW direction of servomotor. This limit is effective if the TCCW (torque limit in CCW direction) is on by DI input. When limit is effective, the actual torque limitation will take the minimum absolute value from the maximum overload capacity of the servo driver, the internal CCW torque limitation and the external CCW torque limitation. |  |  |  |  |  |  |
| F068 | External torque limit in CW direction | -300～0 |  | -100 | % | ALL |
| Set the external torque limitation value in CW direction of servomotor. This limit is effective if the TCW (torque limit in CW direction) is on by DI input. When limit is effective, the actual torque limitation will take the minimum absolute value from the maximum overload capacity of the servo driver, the internal CCW torque limitation and the external CCW torque limitation. |  |  |  |  |  |  |
| F069 | Torque limit in trial running | 0～300 |  | 100 | % | ALL |
| Set the torque limitation value for trial running mode (the speed JOG movement, the keyboard speed adjustment, the demonstration mode). The torque limitation is not related to the rotation direction. It is valid in both directions. The internal and the external torque limitation are still effective. |  |  |  |  |  |  |
| F070 | Alarm level of torque overload in CCW direction | 0～300 |  | 300 | % | ALL |
| Set the overload value of torque in (CCW) direction. This value indicates the percentage of rated torque. When the torque of the servomotor exceeds F070 and the duration is longer than F072, then the servo driver alarms, and the servomotor stops. The number of the alarm is Err29. |  |  |  |  |  |  |
| F071 | Alarm level of torque overload in CW direction | -300～0 |  | -300 | % | ALL |
| Set the overload value of torque in (CW) direction. This value indicates the percentage of rated torque. When the torque of the servomotor exceeds F071 and the duration is longer than P072, then the servo driver alarms, and the servomotor stops. The number of the alarm is Err29. |  |  |  |  |  |  |
| F072 | Detection time for torque overload alarm | 0～10000 |  | 0 | 10ms | ALL |
| Refer to the description of parameter F070 and F071. The torque overload can be shielded if the set value is zero. |  |  |  |  |  |  |
| F075 | Maximum speed limit | 0～5000 |  | 3500 | r/min | ALL |
| Set the maximum speed limitation for the servomotor. The limit is effective in both CCW and CW direction. If the set value exceeds the system maximum allowable speed, the actual speed also can be limited in the maximum speed. |  |  |  |  |  |  |
| F076 | JOG running speed | 0～5000 |  | 100 | r/min | S |
| Set the running speed for JOG operation. |  |  |  |  |  |  |
| F080 | Position out-of-tolerance detection | 0.00～327.67 |  | 4.00 | 圈 | P |
| Set the detection range of position out-of-tolerance. Under position control mode, when the counting value of position deviation counter exceeds the pulses corresponding to this parameter value, the servo driver gives the position out-of-tolerance alarm (Err 4). The unit is one circle. Multiplying the resolution of encoder with the value of this parameter can obtain the total pulse number. For example, the encoder has 2500 lines and the resolution of encoder is 10000. If the parameter value is 4.00, then corresponds to 40000 pulses. |  |  |  |  |  |  |
| F096 | Items of initial display | 0～22 |  | 0 | - | ALL |
| Set the display status on the front panel after turn on the power supply. The meanings of this parameter are： F096; Display item; F096; Display item. 0; Speed of servomotor; 12; Reserved. 1; Original Position command; 13; Reserved. 2; Position command; 14; Digital input DI. 3; Position of servomotor; 15; Digital output DO. 4; Position deviation; 16; Signals of encoder. 5; Torque; 17; Absolute position in one turn. 6; Peak torque; 18; Accumulative load ratio. 7; Current; 19; Brake ratio. 8; Peak current; 20; Control mode. 9; Frequency of input pulse; 21; Number of alarm. 10; Speed command; 22; Reserved. 11; Torque command; -; -. |
| F097 | Neglect inhibition of servo driver | 0～3 |  | 3 | - | ALL |
| The prohibited forward travel (CCWL) and the prohibited reverse travel (CWL) from DI inputs are used for the limit traveling protection. Use normal closed switch as protecting switch. If the input from DI is ON, then the servomotor can move to this direction, or is OFF, cannot move to this direction. If does not use the limit traveling protection, can neglect it by modifying this parameter and does not need the CCWL and CWL wiring. The default value neglects the prohibition, if use this function, please modify this value first. The meanings of this parameter are: F097; Motion inhibition in CW direction(CWL); Motion inhibition in CCW direction(CCWL). 0; Use; Use. 1; Use; Neglect. 2; Neglect; Use. 3; Neglect; Neglect. Use: When input signal is ON, the servomotor can move to this direction; When OFF the servomotor cannot move to this direction. Neglect: The servomotor can move to this direction, and the prohibition signal does not have the function, therefore can disconnect this signal. |
| F098 | Forced enable | 0~1 |  | 3 | - | ALL |
| The meanings of this parameter are: 0：The enable signal SON comes from inputs by DI; 1：The enable signal comes from internal software. |  |  |  |  |  |  |

5.1.2 Parameters of section 1

| Para- meter | Name | Range | Default value | Unit | Usage |
| --- | --- | --- | --- | --- | --- |
| F100 | Function of digital input DI1 | -21～21 | 1 | - | ALL |
| The function plan of digital input DI1: the absolute value of the parameter expresses functions; the symbolic expresses the logic. Refer to section 5.5 for the functions. The symbolic expresses the input logic. Positive number expresses positive logic and the negative number express the negative logic. ON is effective, OFF is invalid: Parameter; DI input signal; DI Result. Positive number; Turn off; OFF. Turn on; ON. Negative number; Turn off; ON. Turn on; OFF. If the same function is set to multiple input channels, the function result is logic or relation. For example F100 and F101 are set to 1 (the SON function), then if any of DI1, DI2 is ON, the SON is effective. The input function which is not selected by parameter F100~F104, namely the undefined function, result is OFF (invalid). |
| F101 | Function of digital input DI2 | -21～21 | 2 | - | ALL |
| The function plan of digital input DI2. Refer to the description of parameter F100. |  |  |  |  |  |
| F102 | Function of digital input DI3 | -21～21 | 3 | - | ALL |
| The function plan of digital input DI3. Refer to the description of parameter F100. |  |  |  |  |  |
| F103 | Function of digital input DI4 | -21～21 | 4 | - | ALL |
| The function plan of digital input DI4. Refer to the description of parameter F100. |  |  |  |  |  |
| F104 | Function of digital input DI5 | -21～21 | 20 | - | ALL |
| The function plan of digital input DI5. Refer to the description of parameter F100. |  |  |  |  |  |
| F110 | Filter of digital input DI1 | 0.1～100.0 | 2.0 | ms | ALL |
| This is the time-constant of DI1 input digital filter. The smaller the value, the quicker the signal response; the bigger the value, the slower the signal responses, but filtering ability of noise is stronger. |  |  |  |  |  |
| F111 | Filter of digital input DI2 | 0.1～100.0 | 2.0 | ms | ALL |
| This is the time-constant of DI2 input digital filter. Refer to the description of parameter F110. |  |  |  |  |  |
| F112 | Filter of digital input DI3 | 0.1～100.0 | 2.0 | ms | ALL |
| This is the time-constant of DI3 input digital filter. Refer to the description of parameter F110. |  |  |  |  |  |
| F113 | Filter of digital input DI4 | 0.1～100.0 | 2.0 | ms | ALL |
| This is the time-constant of DI4 input digital filter. Refer to the description of parameter F110. |  |  |  |  |  |
| F114 | Filter of digital input DI5 | 0.1～100.0 | 2.0 | ms | ALL |
| This is the time-constant of DI5 input digital filter. Refer to the description of parameter F110. |  |  |  |  |  |
| F130 | Function of digital output DO1 | -12～12 | 2 | Pulse | ALL |
| The function plan of digital output DO1: The absolute value of the parameter expresses functions; the symbol expresses the logic, refer to section 5.6 for the functions. '0' is forcing OFF, '1' is forcing ON. The symbol indicates the output logic; the positive number expresses the positive logic and the negative number expresses the negative logic: Parameter value; Function; DO output signal. Positive number; ON; Turn on. OFF; Turn off. Negative number; ON; Turn off. OFF; Turn on. |
| F131 | Function of digital output DO2 | -12～12 | 3 | Pulse | ALL |
| This is the function plan of digital output DO2. Refer to the description of parameter F130. |  |  |  |  |  |
| F132 | Function of digital output DO3 | -12～12 | 8 | r/min | ALL |
| This is the function plan of digital output DO3. Refer to the description of parameter F130. |  |  |  |  |  |
| F150 | Range for positioning completion | 0～32767 | 10 | r/min | P |
| Set the pulse range for positioning completion under the position control mode. When the pulse number in the position deviation counter is smaller than or equals to this set value, the digital output DO COIN is ON (positioning completion), otherwise is OFF. The comparator has return difference function set by parameter F151. |  |  |  |  |  |
| F151 | Return difference for positioning completion | 0～32767 | 5 | - | P |
| Refer to the description of parameter F150. |  |  |  |  |  |
| F154 | Arrival speed | -5000～5000 | 500 | - | ALL |
| When the servomotor speed exceeds this parameter, the digital output DO ASP (speed arrives) is ON, otherwise is OFF. The comparator has return difference function set by parameter F155. The polarity setting function: F156; F154; Comparator. 0; >0; Detect CCW or CW speed. 1; >0; Detect CCW speed only. <0; Detect CW speed only. |
| F155 | Return difference of arrival speed | 0～5000 | 30 | r/min | ALL |
| Refer to the description of parameter F154. |  |  |  |  |  |
| F156 | Polarity of arrival speed | 0～1 | 0 | ms | ALL |
| Refer to the description of parameter F154. |  |  |  |  |  |
| F163 | Position deviation clearance mode | 0～1 | 0 | ms | P |
| In the position control mode, use the CLR input signal (clear position deviation) from DI to clear the position deviation counter. The meaning of this parameter are:( at the time when the position deviation clearance occurs) 0: The high level of CLR ON. 1: The rising edge of CLR ON (the moment from OFF to ON). |  |  |  |  |  |
| F165 | Range for static check of the servomotor | 0～1000 | 5 | r/min | ALL |
| Use this parameter to check the servomotor to be static. If the speed of the servomotor is lower than the parameter value then consider the servomotor static. Only applied to the timing chart judgment of the electromagnetic brake. |  |  |  |  |  |
| F166 | Delay time for electromagnetic brake when servomotor in static state | 0～2000 | 0 | - | ALL |
| Use the electromagnetic brake when the SON is from ON to OFF or alarm occurs in the servo driver. This parameter defines the delay time from the action (the BRK is OFF from DO terminals) of the electromagnetic brake to excitation removal of the servomotor during the servomotor in the static state. The parameter should be no smaller than the delay time of machinery brake. This parameter will make the brake reliable and then turns off the servomotor excitation to guarantee against the small displacement of the servomotor or depreciation of the work piece. For the corresponding timing chart please refer to section 4.7.3. |  |  |  |  |  |
| F167 | Waiting time for electromagnetic brake when servomotor in motion | 0～2000 | 500 | - | ALL |
| Use the electromagnetic brake when the SON is from ON to OFF or alarm occurs in the servo driver. This parameter defines the delay time from excitation removal of the servomotor to the action (the BRK is OFF from DO terminals) of the electromagnetic brake during the servomotor in motion state. This parameter will make the servomotor decelerate from high speed down to low speed and then apply the brake to avoid damaging of the braker. The actual action time will take the smaller value from the parameter F167 and the time in which the servomotor decelerates to the F168 value. For the corresponding timing chart please refer to section 4.7.3. |  |  |  |  |  |
| F168 | Action speed for electromagnetic brake when servomotor in motion | 0～3000 | 100 | - | ALL |
| For details please refer to the description of parameter F167. |  |  |  |  |  |

5.2 DI function description in detail

| Ordinal | Symbol | DI Function | Function description |
| --- | --- | --- | --- |
| 0 | NULL | No function | The input condition has no influence on the system. |
| 1 | SON | Servo enable | OFF：servo driver not enabled, servomotor not excited; ON：servo driver enabled, servomotor excited. |
| 2 | ARST | Alarm clearance | When alarm occurs and the alarm has permission to clear, then the rising edge (from OFF to ON) of input signal ARST will clear the alarm. Attention: only a part of alarm can be cleared. |
| 3 | CCWL | CCW drive inhibition | OFF：Inhibit CCW running; ON ：Allow CCW running. The function is for protection of the mechanical traveling limit, and is controlled by the parameter P097. Pay attention to that the F097 default value neglects this function, therefore needs to modify F097 if this function is used: F097; Description. 0; Use CCW prohibition function and must connect the normally closed contact of the limit switch.. 2. 1; Neglect CCW prohibition function, servomotor can operation in the positive direction, the signal is invalid and need not to be connected.. 3(Default). |
| 4 | CWL | CW drive inhibition | OFF：Inhibit CW running; ON ：Allow CW running. The function is for protection of the mechanical traveling limit, and is controlled by the parameter F097. Pay attention to that the F097 default value neglects this function, therefore needs to modify F097 if this function is used: F097; Description. 0; Use CW prohibition function and must connect the normally closed contact of the limit switch.. 2. 1; Neglect CW prohibition function, servomotor can operation in the positive direction, the signal is invalid and need not to be connected.. 3(Default). |
| 5 | TCCW | CCW torque limitation | OFF：Torque is not limited by parameter F067 in CCW direction; ON ：Torque is limited by parameter F067 in CCW direction. Attention: no matter the TCCW is effective or not, the torque is also limited by the parameter F065 in CCW direction. |
| 6 | TCW | CW torque limitation | OFF：Torque is not limited by parameter F068 in CW direction; ON ：Torque is limited by parameter F068 in CW direction. Attention: no matter the TCW is effective or not, the torque is also limited by the parameter F066 in CW direction. |
| 15 | EMG | Emergency stop | OFF：Allow the servo driver to work; ON ：Servo driver stops; removes the main current and the excitation of servomotor. |
| 18 | GEAR1 | Electronic gear selection 1 | Select electronic gear for command pulse by the combination of GEAR1 and GEAR2 1~4: GERA2; GERA1; Numerator of electronic gear N. 0; 0; 1stnumerator(parameterF029). 0; 1; 2ndnumerator(parameterF031). 1; 0; 3rdnumerator(parameterF032). 0; 1; 4thnumerator(parameterF033). Note: 0 represents OFF, 1 represents ON. |
| 19 | GEAR2 | Electronic gear selection 2 |  |
| 20 | CLR | Position deviation clearance | Eliminate the position deviation counter; The elimination mode is selected by the parameter F163; The elimination of position deviation occurs in the moment: P163=0：CLR ON Level: P163=1：CLR Rising edge (from OFF to ON). |
| 21 | INH | Pulse input inhibition | OFF：Allow position command pulse to go through: ON ：Position command pulse is inhibited. |

5.3 DO function description in detail

| Ordinal | Symbol | DO Function | Function description |
| --- | --- | --- | --- |
| 0 | OFF | Always invalid | Forced output OFF. |
| 1 | ON | Always valid | Forced output ON. |
| 2 | RDY | Servo ready | OFF：Servo main power supply is off; Or alarm occurs; ON ：Servo main power supply is normal，no alarm occurs. |
| 3 | ALM | Alarm | OFF：Alarm occurs； ON ：No alarm occurs |
| 5 | COIN | Positioning completion | In position control mode OFF：Position deviation is bigger than parameter F150; ON ：Position deviation is smaller than parameter F150. |
| 6 | ASP | Arrival speed | OFF：Servomotor speed is lower than parameter F154; ON ：Servomotor speed is higher than parameter F154. Can set polarity function，for details please refer to the description of parameter F154. |
| 8 | BRK | Electromagnetic brake | OFF：Electromagnetic brake brakes; ON ：Electromagnetic brake releases the brake. |
| 11 | TRQL | Torque under limitation | OFF：Servomotor torque has not reached the limit value; ON ：Servomotor torque has reached the limit value. Torque limitation is set by parameter F064. |

Section VI Alarm table

6.1 Alarm table

| Alarm code | Alarm name | Alarm content | Alarm clear |
| --- | --- | --- | --- |
| Err-- | No alarm | Normal operation | - |
| Err 1 | Over speed | Servomotor speed exceeds the speed limit | No |
| Err 2 | Over voltage of the main circuit | The voltage of the main power supply exceeds the specified value. | No |
| Err 4 | Position deviation exceeds the limit value | The counter of position deviation exceeds the set limit value. | YES |
| Err 7 | Drive inhibition abnormal | CCWL、CWL the inputs of drive inhibition are not effective. | YES |
| Err 8 | Overflow of position deviation counter | The absolute value of position deviation counter exceeds 230 | YES |
| Err 9 | Encoder signal fault | Lack of the signals of encoder | No |
| Err11 | Power module fault | Power model fault occurs | No |
| Err12 | Over current | Excessive current of servomotor | No |
| Err13 | Overload | Overload of servomotor | No |
| Err14 | Overload of brake peak power | Excessive instantaneous load in short brake time | No |
| Err15 | Encoder counter fault | Encoder counter is abnormal | No |
| Err16 | Over-heat of servomotor | The heat load of servomotor exceeds the set value (I2t detection) | No |
| Err17 | Overload of brake average power | Excessive average load in brake time | No |
| Err18 | Overload of power module | Excessive average output load of power module | No |
| Err20 | EEPROM fault | EEPROM fault occurs when read or white | No |
| Err21 | Logic circuit fault | Logic circuit fault outside DSP | No |
| Err23 | AD conversion fault | Circuit or current sensor fault | No |
| Err24 | Under voltage of control power supply | The LDO fault of control circuit | No |
| Err29 | Over-torque alarm | The load of servomotor exceeds the set value and lasting time | YES |
| Err30 | Encoder Z signal lost | Encoder Z signal is not appeared. | No |
| Err31 | Encoder UVW signal fault | The UVW signal fault or pole number does not match with the servomotor | No |
| Err32 | Illegal code of encoder UVW signals | UVW signals are all high level or low level | No |
| Err33 | Wire-saving encoder signal fault | No high resistance in the timing chart upon power on. | No |

6.2 The cause and the management of the alarm

| Alarm code | Potential cause | Check up | Management |
| --- | --- | --- | --- |
| Err 1 Over speed | Servomotor U、V、W connection is not right | Check U、V、W wiring | Correct U、V、W wiring. The U、V、W must connect with servo driver terminal U、V、W correspondently. |
|  | Speed overshoot | Check operation status and the parameters. | Adjust servo gain to reduce the overshoot; In speed control mode can increase acc./dec. time. |
|  | Encoder wiring fault | Check the encoder wiring | Correct wiring |
| Err 2 Main circuit over-voltage | Excessive input AC power supply | Check the voltage of power supply | Use correct power supply according to the specification. |
|  | Regeneration fault | Regenerative resistor / IGBT damaged; Or connection circuit is open. | Maintenance |
|  | Excessive regeneration energy | Check the regeneration load factor | Reduce the starting and stopping frequency. Increase acc./ dec. set time. Reduce the torque limit. Reduce the load inertia. Replace the servo driver and servomotor with higer power levels. |
| Err 4 Excessive position deviation | Servomotor U、V、W connection fault | Check U、V、W wiring | Correct U、V、W wiring. The U、V、W must connect with servo driver terminal U、V、W correspondently. |
|  | Encoder zero point variation | Check encoder zero point | Install the encoder again and adjust the zero point |
|  | Encoder wiring fault | Check encoder wiring | Correct wiring |
|  | Servomotor is blocked | Check the servomotor shaft and its mechanical connection | Maintenance |
|  | Excessive command pulse frequency | Check input frequency and the parameter of division multiplication | Slow down the input frequency. Adjust the parameter of division multiplication. |
|  | Deficient position loop gain | Check the parameter F009 | Increase the gain of position loop. |
|  | Deficient range of excess position deviation check | Check the parameter F079 | Increase the value of parameter P079. |
|  | Deficient torque | Check torque | Increase the torque limit. Increase smooth filtering time for position command. Reduce load. Replace the servo driver and servomotor with larger power levels. |
| Err 7 Drive inhibition abnormities | The CCWL / CWL drive inhibition input is invalid when servo is on | Check CCWL、CWL wiring | Correct input CCWL、CWL signal. If not use CCWL、CWL signal can shield it by setting parameter F097. |
| Err 8 Overflow of position deviation counter | The servomotor is blocked | Check the servomotor shaft and its mechanical connection | Maintenance |
|  | The command pulse is abnormal | Check command pulse | - |
| Err 9 Encoder signal fault | Encoder wiring fault | Check the encoder wiring | Correct wiring |
|  | Encoder cable / connector is bad | Check cable & connector | Replace cable & connector. |
|  | Servomotor type setting is not correct | Check servomotor type | Reset the servomotor type |
|  | Encoder is damaged | Check the encoder | Replace the encoder |
| Err 11 IGBT module fault | Short-circuit at drive output (U、V、W) | Check U、V、W wiring | Repair or replace the short- circuited wiring. |
|  | Motor winding insulation is damaged | Check the servomotor | Replace the servomotor |
|  | Servo driver is damaged | Check the servo driver | Known that the servomotor has no fault, and then turn on the power supply again, if the alarm still exists, the servo driver may be damaged possibly. Replace the servo driver. |
|  | Imperfect earth | Check the ground wiring | Ground correctly. |
|  | Suffer from interference | Check interference source | Add line filter; keep away from interference source |
| Err 12 Over-current | Short-circuit of drive output (U、V、W) | Check wiring connections of U、V、W | Correct wiring of U、V、W |
|  | Motor winding insulation is damaged | Check the servomotor | Replace the servomotor |
|  | Servo driver is damaged | Check the servo driver | Known that the servomotor has no fault, and then turn on the power supply again, if the alarm still exists, the servo driver may be damaged possibly. Replace the servo driver. |
| Err 13 Over-load | Excess the rated load for continuous operation | Check the load rate | Reduce load or replace the servo driver with larger power levels. |
|  | System unstable | Check if oscillation during servomotor in running | Reduce gain of the system. |
|  | Acc./dec. is too fast | Check the smoothness during servomotor in running | Increase acc./dec. set time |
|  | Encoder zero point variation | Check the encoder zero point | Install the encoder again and adjust the zero point. |
| Err 14 Overload of brake peak power | Excessive input AC power supply | Check the voltage of power supply | Use correct power supply according to the specification.使 |
|  | Regeneration fault | Regenerative brake resistor / IGBT damaged; Or connection circuit is open. | Maintenance |
|  | Excessive regeneration energy | Check the regeneration load rate | Reduce the starting and stopping frequency. Increase acc./dec. set time Replace the servo driver and servomotor with larger power levels. |
| Err 15 Encoder counter fault | Encoder wiring fault | Check encoder wiring | Correct wiring include shield wire |
|  | Imperfect earth | Check the ground wiring | Ground correctly. |
|  | Suffer from interference | Check interference source | Keep away from interference source. |
|  | Encoder has problem | Check the line number and pole number Check encoder Z signal Encoder damaged | Replace the encoder. |
| Err 16 Motor over-heat | Excess the rated load for continuous operation | Check the load rate and the temperature rise of motor | Reduce load or replace the servo driver with larger power levels. |
|  | Encoder zero point variation | Check the encoder zero point | Reinstall the encoder and adjust the zero point. |
| Err 17 Overload of average brake power | Excessive voltage of input AC power supply | Check the voltage of power supply | Use correct power supply according to specifications. |
|  | Excessive regeneration energy | Check the brake load rate | Reduce the starting and stopping frequency. Reduce value of torque limit. Reduce load inertia. Increase acc./dec. set time Replace the servo driver and servomotor with larger power levels. |
| Err 18 Overload of power module | Excess the rated load for continuous operation | Check current | Reduce load or replace the servo driver with larger power levels. |
|  | Encoder zero point variation | Check the encoder zero point | Reinstall the encoder and adjust the zero point. |
| Err 20 EEPROM fault | EEPROM chip is damaged | Turn on the power again and check | If the fault still exists, then replace the servo driver. |
| Err 21 Logic circuit fault | Control circuit fault | Turn on the power again and check | If the fault still exists, then replace the servo driver. |
| Err 23 AD conversion fault | Current sensor and connector fault | Check the main circuit | Replace the servo driver. |
|  | AD converter and analog amplifier fault | Check the control circuit | Replace the servo driver. |
| Err 24 Under voltage of control power supply | Control circuit LDO fault | Check the power of control board | Replace the servo driver. |
| Err 29 Torque overload alarm | Unexpected big load occurs | Check load condition | Correctly adjust the load. |
|  | Parameter P070, P071, P072 setting is not reasonable | Check the parameters | Correctly adjust the load. |
| Err 30 Encoder Z signal lost | Encoder has problem | Check the encoder Z signal | Replace the encoder. |
|  | Encoder cable/ connector has problem | Check cable and connector | Replace the cable and connector. |
|  | The interface circuit of the servo driver has faul. | Check the control circuit | Replace the servo driver. |
| Err 31 Encoder UVW signal fault | Encoder has problem | Check the line number and pole number Check the encoder UVW signals Encoder damaged | Replace the encoder. |
|  | Encoder wiring fault | Check the encoder wiring | Correct wiring include shield wire. |
| Err 32 Encoder UVW signal illegal coding | Encoder has problem | Check the encoder UVW signals | Replace the encoder |
|  | Encoder wiring fault | Check the encoder wiring | Correct wiring include shield wire. |
| Err 33 Wire saving encoder fault | Encoder has problem | Check the encoder signals | Replace the encoder |
|  | Servomotor set type is not correct | Check the servomotor type; confirm that the servomotor is adapted with the wire saving encoder. | Reset the servomotor type . |