IHSV

Integrated ac servo motor manual

Introduction

All contents of this manual, copyright owned by Shenzhen Just Motion Control

Electromechanics Co.,Ltd., shall not be arbitrarily reproduced, copied, transcribed

without permission. This manual does not contain any forms of guarantee,

standpoint statement, or hint. Shenzhen Just Motion Control and its employees will

not take any responsibility for the loss caused by direct or indirect leaking

information mentioned in this Manual. In addition, products information in this

manual is for reference only, we are sorry for not offering update if it is improved.Shenzhen Just Motion Control Electromechanics Co.,ltd

0755-26509689

Shenzhen Just Motion Control Electro-mechanics Co., Ltd

TEL:+86-0755-26509689

FAX:+86-0755-26509289

www.jmc-motion.com

Email: info@jmc-motion.com

Address: Floor2, Building A, Hongwei Industrial Zone No.6,

Liuxian 3rd Road, Shenzhen. China

Shenzhen Just Motion Control Electro-mechanics Co., Ltd

Version Editor Verifier

V2.0 R&D R&D

Contents

1. Overview ............................................................................................................................ 2

2. Features.............................................................................................................................. 4

3. Technical index ................................................................................................................ 5

4. Ports Introduction .............................................................................................................. 6

2Shenzhen Just Motion Control Electromechanics Co.,ltd

0755-26509689

4.1 ALM signal output ports .................................................................................................. 6

4.2 Control Signal Input Ports ............................................................................................... 7

4.3 Power Interface Ports ...................................................................................................... 8

5. Parameter and function .................................................................................................. 8

5.1 List of parameters............................................................................................................ 8

5.2 Parameter parsing instructions ....................................................................................... 16

5.2.1 P00-xx Motor and Drive Parameters ........................................................................... 16

5.2.2 P01-xx Main control parameters ................................................................................. 18

5.2.3 P02-xx Gain parameters ........................................................................................... 20

5.2.4 P03-xx Position parameters......................................................................................... 26

5.2.5 P04-xx Speed parameter ............................................................................................. 27

5.2.6 P05-xx 8/5000Torque parameters ................................................................................ 28

5.2.7 P06-xx I/O parameter ................................................................................................. 28

5.2.8 P08-xx Advanced function parameters ........................................................................ 30

5.3

List of monitored items ................................................................................................ 31

6. Fault Analysis and Processing ....................................................................................... 33

6.1 Fault Alarm Information Form ....................................................................................... 33

6.2 Causes and Disposal of Fault Alarm............................................................................... 36

7.

Control signal wiring..................................................................................................... 39

7.1 Control signal single terminal coanode wiring ................................................................ 39

7.2 Control signal single terminal cocathode wiring ............................................................. 41

7.3 Control signal differential wiring mode .......................................................................... 42

7.4 232Serial communication wiring ................................................................................... 43

7.5 Control signal timing diagram........................................................................................ 44

7.6 Chart of torque characteristic of servo motor speed......................................................... 45

8. Subdivision code switch settings ...................................................................................... 49

8.1 Subdivision settings ......................................................................................................... 50

8.2 Enter Edge Settings ....................................................................................................... 51

8.3 logical Direction Settings .............................................................................................. 51

3Shenzhen Just Motion Control Electromechanics Co.,ltd

0755-26509689

9. Error alarm and LED light flicker times ......................................................................... 50

10. Installation dimensions(mm) .......................................................................................... 51

11. Frequently Asked Questions and Fault Analysis .......................................................... 55

11.1 Power's not on. ............................................................................................................... 55

11.2Turn on the red light. ....................................................................................................... 55

11.3 Run, turn a small angle, turn on the red light. .................................................................. 55

11.4 After the pulse is entered, it does not rotate. .................................................................... 55

12. IHSV42/57/60/86-XX Parameter modification step ....................................................... 56

1. Overview

IHSV42/57/60 / 86-XX Integrated AC servo drive motor integrates AC servo drive into servo

motor. The two are perfectly integrated and use vector control designed and produced by DSP. It

has the characteristics of low cost, full closed-loop, full number, low heat, small vibration and

fast response. Includes three adjustable feedback loop controls(position loop, speed loop, and

current loop). Performance stability, is a very high cost performance of sports control products.

2、

、 Features

2.1 Multiple pulse input modes Pulse + Direction

2.2 Optically coupled isolation servo reset input interface ERC

2.3 Current loop bandwidth:(-3 dB) 2 KHz(typical value)

4Shenzhen Just Motion Control Electromechanics Co.,ltd

0755-26509689

2.4 Speed loop bandwidth: 500 Hz(typical value)

2.5 Position loop bandwidth: 200 Hz(typical value)

2.6 Motor end orthogonal encoder input interface: differential input(26LS32)

2.7 Download parameters via PC or text display with RS232C interface

2.8 Users can choose to subdivide through external dialing switches, or they can use software

Define subdivision

2.9 Overflow, I2T, Overpressure, Speeding, Ultra-Poor Protection

2.10 A green light indicates operation, a red light indicates protection or offline

3、

、 Technological Index

42base

Input voltage(VDC)

57base

60base

52 78 100 140 180 200 400

W W W W W W W

24V

36V

36V

48V

Max pulse frequency 200K

Default communication rate 9.6Kbps(Additional interface required)

Protection ▸Overload I2t Current Action Value 300 % 3S

Environment

Environment Avoid dust, oil fog and corrosive gases

Working 0~+70°C

temperature

Specifications

Storage

-20°C~+80°C

temperature

5

86base

440 660

W W

48V

72VShenzhen Just Motion Control Electromechanics Co.,ltd

0755-26509689

Humidity 40~90%RH

Cooling Natural cooling or forced air cooling

method

4、

、 Ports Introduction

4.1 ALM signal output ports

Symbol

Name

Instruction

1

ALM- Alarm output -

ALM+ Alarm output +

PED- Arrive position output-

PED+ Arrive position output+

6

11

5

15

10

+

6

-Shenzhen Just Motion Control Electromechanics Co.,ltd

0755-26509689

4.2 Control Signal Input Ports

Symbol

Function

Remark

Enabling signal: This input signal is used to enable or

ENA-

prohibit; In addition, it can be used to clear the drive alarm. Low 0 ~ 0.5 V valid

When ENA + is connected to +5 V, ENA-is connected to

low power, the drive will cut off the current of each phase

of the motor so that the motor is in a free state, where the

pulse is not responded and the alarm can be cleared; When

ENA+

this function is not required, the energy signal end is

High 4 ~ 5V valid

suspended.

DIR-

Direction signal: high/low level signal. In order to ensure

Low 0 ~ 0.5 V valid

the reliable direction change of the motor, the direction

DIR+

PUL-

PUL+

signal should be established before the pulse signal at least

6us.

High 4 ~ 5V valid

Pulse control signal: Pulse rising edge is effective. In order Low 0 ~ 0.5 V valid

to reliably respond to pulse signal, the pulse width should

be greater than 2.5 us.

7

High 4 ~ 5V validShenzhen Just Motion Control Electromechanics Co.,ltd

0755-26509689

4.3 Power port

Identification

Symbol Name

DC+ Power+

20VDC-80VDC(sel

ect voltage and

power supply

Power input

terminal

Remark

according to the

GND

Power -

corresponding type

of motor's technical

index)

5、

、 Parameters and function

5.1 Parameters list

P00-xx Represents motor and drive parameters

P01-xx Main control parameters

P02-xx Represents the gain class parameter

P03-xx Represents the position parameter

P04-xx Represents the speed parameter

P05-xx Represent the torque speed

P06-xx Represent I/O parameter

P08-xx Represents an advanced function parameter

8Shenzhen Just Motion Control Electromechanics Co.,ltd

0755-26509689

Model

Param

eter

code Name

P00-00 Motor No.

P00-01

P00-02

P00-03

P00-04

Motor

P00-05

Motor rated

speed

Motor rated

torque

Motor rated

current

Motor inertia

Pole number

of motor

Setting

range

Factor

y

setting

Unit

Setting

way

Stop

0-65535 --- 1-6000 --- rpm

0.01-655.35 --- N.M

0.01-655.35 --- A

0.01-655.35 --- kg.cm2

1-31 --- 0-65535 --- 0-65535 --- 0-360 --- 1degree

0-360 --- 1degree

0-360 --- 1degree

0-360 --- 1degree

Setting

Stop

Setting

Stop

Setting

Stop

Setting

Stop

Setting

Opposit Stop

e pole Setting

Effective

time

Re-power

Re-power

Re-power

Re-power

Re-power

Re-power

and

Number of

driver

parame

ters

P00-10

lines in

incremental

Stop

Setting

Re-power

encoder

Incremental

P00-11

Motor

P00-12

parame

ters

Stop

Setting

pulse angle

and

driver

encoder Z

P00-13

P00-14

P00-15

Initial angle

of rotor 1

Initial angle

of rotor2

Initial angle

of rotor3

Initial angle

of rotor4

9

Stop

Setting

Stop

Setting

Stop

Setting

Stop

Setting

Re-power

Re-power

Re-power

Re-power

Re-powerShenzhen Just Motion Control Electromechanics Co.,ltd

0755-26509689

P00-16

P00-17

Initial angle

of rotor5

Initial angle

of rotor6

0-360 --- 1degree

0-360 --- 1degree

0-3 2 ---

0-255 1 ---

0-7 7 ---

0-3 1 ---

0-100 0 1ms

0-300 0 1V

0-2 0 ---

0-2 1 ---

RS232

P00-21

communicate

baud rate

P00-23

Slave address

Modbus

P00-24

=communicat

e baud rate

P00-25

Check mode

Stop

Setting

Stop

Setting

Stop

Setting

Stop

Setting

Stop

Setting

Stop

Setting

Re-power

Re-power

Re-power

Re-power

Re-power

Re-power

ModbusCom

P00-26

munication

response

Stop

Setting

Re-power

delay

Overvoltage

P00-42

protection

threshold

P01-01

Control mode

setting

Real time

Main

control

P01-02

automatic

adjustment

parame mode

ters Real time

P01-03

automatic

adjustment of

Stop

Setting

Stop Effective

Setting immediately

Run

Setting

s

Run

0-31

13

---

Setting

s

rigidity setting

10

Re-power

Effective

immediately

Effective

immediatelyShenzhen Just Motion Control Electromechanics Co.,ltd

0755-26509689

The ratio of

P01-04

moment of

Run

0-100.00

1

1times

inertia

Setting

s

Effective

immediately

Brake

command -

P01-30

servo OFF

delay time

Run

0-255

100

1ms

Setting

s

(brake open

Effective

immediately

delay)

Speed limit

P01-31

value of brake

command

Run

0-3000

100

1rpm

s

output

Servo OFF

P01-32

brake

command

Run

0-255

100

1ms

P02-00

Gain

P02-01

parame

ter

control gain 1

Position

control gain 2

Run

0-3000.0

48.0

1/S

feedforward

Run

0-3000.0

57.0

1/S

Run

0-100.0

30.0

1.0%

smoothing

Run

0-64.00

0.5

1ms

Speed

Setting

s

constant

P02-10

Setting

s

Speed

P02-04

Setting

s

gain

feedforward

Setting

s

Speed

P02-03

Setting

s

waiting time

Position

Setting

1.0-2000.0

11

27.0

1Hz

Run

Effective

immediately

Effective

immediately

Effective

immediately

Effective

immediately

Effective

immediately

Effective

immediately

EffectiveShenzhen Just Motion Control Electromechanics Co.,ltd

0755-26509689

Gain

proportional Setting

gain1 s

immediately

parame

ter

P02-11

Speed integral

constant 1

Run

0.1-1000.0

10.0

1ms

Setting

s

Effective

immediately

Pseudo

differential

P02-12

feedforward

Run

0-100.0

100.0

1.0%

control

Setting

s

Effective

immediately

coefficient1

Speed

P02-13

P02-14

proportional

Run

1.0-2000.0

27.0

1Hz

Setting

gain 2 s

Velocity Run

integral

0.1-1000.0

1000.0

1ms

constant2

Setting

s

Effective

immediately

Effective

immediately

Pseudo

differential

P02-15

feedforward

Run

0-100.0

100.0

1.0%

control

Setting

s

Effective

immediately

coefficient2

Torque

P02-19

feedforward

Run

0-30000

0

1.0%

gain

s

Torque feed

P02-20

forward

smoothing

Run

0-64.00

0.8

1ms

Gain

Setting

s

constant

P02-30

Setting

0-10

0

12

---

Run

Effective

immediately

Effective

immediately

EffectiveShenzhen Just Motion Control Electromechanics Co.,ltd

0755-26509689

switching Setting

mode s

Gain

P02-31

P02-32

switching

Run

0-20000

800

---

level s

Gain Run

switching

0-20000

100

---

delay

P02-34

switching

Run

0-1000.0

10.0

1ms

s

Position gain Run

switching

0-1000.0

10.0

1ms

Mode switch

level

P02-51

instruction is

Run

0-20000

10000

---

Run

-100.0-100.0

0

1.0%

s

Forward Run

torque

-100.0-100.0

0

1.0%

torque

Run

-100.0-100.0

0

1.0%

0-1 0 ---

0-1 0 ---

source

locatio

The command

pulse is

Setting

s

Location

command

Setting

s

compensation

P03-03

Setting

added

Reverse

P03-00

Setting

s

compensation

P02-52

Setting

s

The torque

P02-50

Setting

delay

time

P02-41

Setting

s

Gain

P02-33

Setting

13

immediately

Effective

immediately

Effective

immediately

Effective

immediately

Effective

immediately

Effective

immediately

Effective

immediately

Effective

immediately

Effective

immediately

Stop Effective

Setting immediately

Stop Effective

Setting immediatelyShenzhen Just Motion Control Electromechanics Co.,ltd

0755-26509689

n

param

reversed

P03-04

eter

Position pulse

filtering

Run

0-3

2

---

gs

Positioning

P03-05

completes the

judgment

Run

0-2

1

---

Location

Completion

0-65535

30

range

Encoder

unit

Number of

P03-09

command

pulses for one

0-65535

4000

Pulse

4000

---

of electronic

4000

immediately

Settin

Re-power

Settin

Re-power

---

Settin

Re-power

gs

Position

deviation too

Effective

Run

1-65535

gear 1

P03-15

immediately

gs

The

denominator

gs

Effective

Run

1-65535

gear 1

P03-11

Settin

immediately

gs

Molecular of

electronic

Run

Effective

Run

motor rotation

P03-10

Settin

gs

condition

P03-06

Settin

0-65535

0

large setting

Instructi Run

on Settin

unit*10 gs

Effective

immediately

Position

instruction

P03-16

smoothing

Run

0-1000.0

0

1ms

filter time

Settin

gs

Effective

immediately

constant

speed

P04-00

Speed

0-1

1

14

---

Stop

EffectiveShenzhen Just Motion Control Electromechanics Co.,ltd

0755-26509689

param

eter

P04-02

P04-05

P04-06

P04-07

command Settin

source g

Digital speed

given value

Speed alarm

value

Forward

speed limit

Reverse speed

limit

Run

-6000—6000

0

1rpm

gs

Run

0-6500

6400

1rpm

detection

Settin

gs

Run

0-6000

5000

1rpm

Settin

gs

Run

0-6000

-5000

1rpm

Settin

gs

Zero speed

P04-10

Settin

Run

0-200.0

40

1rpm

value

Settin

gs

Run

P04-14

P04-15

Speed time

Deceleration

time

0-10000

500

0-10000

Settin

1ms/100 gs

0rpm Run

500

Settin

gs

Internal

torque

P05-10

torque limit

Run

0-300.0

200.0

1.0%

Internal

P05-11

torque limit

Run

0-300.0

200.0

1.0%

value

I/O

P06-00

Enable input

Settin

gs

value

param

eter

positive

Settin

gs

0-4

1

15

---

Run

immediately

Effective

immediately

Effective

immediately

Effective

immediately

Effective

immediately

Effective

immediately

Effective

immediately

Effective

immediately

Effective

immediately

Effective

immediately

Re-powerShenzhen Just Motion Control Electromechanics Co.,ltd

0755-26509689

Param port effective Settin

eter level gs

Alarm output

P06-20

P06-22

port effective

Run

0-1

1

---

level gs

Output port in Run

place valid

0/1

1

---

level

constant of

Run

0-25.00

Feedback

0.8

1ms

Torque

ced

P08-20

Run

command

0-25.00

functi filter constant

on Disturbing

param torque

eters

P08-25

0.84

1ms

Settin

gs

Run

compensation

0-100.0

0

%

Settin

gs

gain

Disturbing

P08-26

Settin

gs

speed

Advan

Settin

Re-power

Re-power

gs

low pass filter

P08-19

Settin

Run

torque filter

0-25.00

0.8

1ms

time constant

Settin

gs

Effective

immediately

Effective

immediately

Effective

immediately

Effective

immediately

5.2 Description of parameter analysis

5.2.1 P00-xx Motor and driver parameters

Parame

ter code

P00-00

P00-01

P00-02

Name Instruction

Motor number Factory set, no need to set 0:P0-01 to P0-17 play a role

Motor rated Setting range :1-6000,Unit:rpm

speed

Motor rated

Factory setting has done ,no need to set

Setting range:0.01-655.35,Unit:N.M

16Shenzhen Just Motion Control Electromechanics Co.,ltd

0755-26509689

P00-03

torque Setting according to the matched motor, factory setting has done

Rated current of Setting range: 0.01-655.35, unit: A

motor According to the motor settings, the factory has been set up.

Setting range: 0.01-655.35, unit: kg.cm2

P00-04

Motor moment

of inertia Setting according to the matched motor , factory setting has

Pole number of Set range: 1-31, unit: opposite pole ,Setting according to the

motor matched motor , factory setting has done

done

P00-05

P00-10

Incremental

encoder number

Incremental

P00-11

encoder Z pulse

Angle

P00-12

P00-13

P00-14

P00-15

P00-16

P00-17

Initial rotor

Setting according to the matched motor , factory setting has

done

Setting according to the matched motor , factory setting has

done

Setting according to the matched motor , factory setting has

Angle 1 done

Initial rotor Setting according to the matched motor , factory setting has

Angle 2 done

Initial rotor Setting according to the matched motor , factory setting has

Angle 3 done

Initial rotor Setting according to the matched motor , factory setting has

Angle 4 done

Initial rotor Setting according to the matched motor , factory setting has

Angle 5 done

Initial rotor

Angle 6

Setting according to the matched motor , factory setting has

done

Setting range :0-3

P00-21

RS232Communi Select the baud rate when communicating with PC0:9600

cation baud rate 1:19200

selection 2:57600

3:115200

17Shenzhen Just Motion Control Electromechanics Co.,ltd

0755-26509689

P00-23

Slave station Setting range:0-255,the default value is 1

address Set up according to equipment requirements

Setting range :0-7,The default value is 2

0:2400

1:4800

P00-24

Modbus 2:9600

Communication 3:19200

baud rate 4:38400

5:57600

6:115200

7:25600

Setting range: 0-3, default value : 1

0: no check, two stop bits

P00-25

Check way

1: even check, 1 stop bit

2: odd check, 1 stop bit

3: no check, 1 stop bit

Modbus

P00-26

Communication

Setting range: 0-100, Unit ms

response delay

Overvoltage

P00-42

protection

Setting range: 0-300, unit V

threshold

5.2.2 P01-xx Master control parameter

Parame

ter

name

Instruction

code

Setting range: 0-6

P01-01

Control mode 0: position control mode

setting 1: speed control mode

2: torque control mode

18Shenzhen Just Motion Control Electromechanics Co.,ltd

0755-26509689

332/5000

Setting range: 0-2

0: manually adjust the rigidity.

1: standard mode automatically adjusts rigidity. In this mode,

parameters p02-00, p02-01, p02-10, p02-11, p02-13, p02-14,

and p08-20 will be set automatically according to the rigidity

level set by p01-03. Manual adjustment of these parameters will

Real time

P01-02

automatic

adjustment mode

not work. The following parameters are set by the user:

P02-03 (speed feed-forward gain), p02-04 (speed feed-forward

smoothing constant).

2: positioning mode automatically adjusts rigidity. In this mode,

parameters p02-00, p02-01, p02-10, p02-11, p02-13, p02-14,

and p08-20 will be set automatically according to the rigidity

level set by p01-03. Manual adjustment of these parameters will

not work. The following parameters will be fixed and cannot be

changed:

P02-03 (speed feedforward gain) : 30.0%

P02-04 (speed feed-forward smoothing constant) : 0.50

P01-03

Adjust the rigid Setting range: 0-31

setting Built-in 32 gain class parameters, when p01-02 set to 1, or 2.

automatically in Can be called directly according to the actual situation, the

real time larger the set value, the stronger the rigidity.

Setting range: 0-100, unit: times

Set the load inertia ratio of the corresponding motor. The setting

P01-04

Moment of inertia

ratio

method is as follows:

P01-04= load inertia/motor moment of inertia

This inertia ratio can use the value identified by AF-J-L

automatic inertia identification and write the value into the

parameter

Brake command -

P01-30

servo OFF delay Setting range: 0-255, unit: ms

time (brake When open the Enable: after the enable instruction is executed,

19Shenzhen Just Motion Control Electromechanics Co.,ltd

0755-26509689

opening delay)

the drive will receive the position instruction after the time of

p01-30.

When turn off the enable when the motor is in a state of rest, the

time after the energy command is executed and the holding gate

is closed until the motor becomes non-energized.

Setting range: 0-3000 Unit RPM

Speed limit value

P01-31

of brake

instruction output

When the motor is in a state of rotation, the motor speed

threshold when the brake output is valid. If it is lower than this

threshold, the brake output command is effective; otherwise, it

will wait for p01-32 time before the gate output command is

effective.

Servo OFF- lock

P01-32

Setting range: 0-255, unit: ms

instruction wait When the motor is in a state of rotation, the maximum waiting

time time for the output of the holding gate.

5.2.3 P02-xx Gain class parameter

Param

eter

code

Name

Instruction

Setting range: 0-3000.0, unit: 1/S

As for the proportional gain of the position loop regulator, the

P02-00

Position control

gain 1

larger the parameter value, the higher the gain ratio, the higher

the stiffness, the smaller the position tracking error and the faster

the response. However, too large parameters can easily cause

vibration and overshoot.

This parameter is for the steady-state response.

Setting range: 0-3000.0, unit: 1/S

As for the proportional gain of the position loop regulator, the

P02-01

Position control

gain 2

larger the parameter value, the higher the gain ratio, the higher

the stiffness, the smaller the position tracking error and the faster

the response. However, too large parameters can easily cause

vibration and overshoot.

20Shenzhen Just Motion Control Electromechanics Co.,ltd

0755-26509689

This parameter is for dynamic response.

Setting range: 0-100.0, unit: 1.0%

The feed forward gain of the speed loop, the larger the

P02-03

Speed feed parameter value, the smaller the system position tracking error,

forward gain and the faster the response. However, if the feedforward gain is

too large, the position loop of the system will be unstable, which

will easily cause overshoot and shock.

P02-04

Speed Setting range: 0-64.00, unit: ms

feedforward This parameter is used to set the speed loop feed-forward filter

smoothing time constant. The larger the value, the larger the filtering effect,

constant but at the same time the phase lag increases.

Setting range: 1.00-2000.0, unit: Hz

The larger the speed proportional gain, the larger the servo

P02-10

Speed stiffness, the faster the speed response, but too large is easy to

proportional gain generate vibration and noise.

1 This parameter value is increased as far as possible under the

condition that the system is not oscillating.

This parameter is for static responses.

Setting range: 1.0-1000.0, unit: ms

Speed regulator integral time constant. The smaller the setting

P02-11

Speed integral

constant 1

value is, the faster the integral velocity is and the higher the

stiffness is.

This parameter value is minimized when the system does not

oscillate.

This parameter is for the steady-state response.

Setting range: 0-100.0, unit: 1.0%

P02-12

Pseudo When the speed loop is set to 100.0%, PI control is adopted and

differential feed the dynamic response is fast. When set to 0, the velocity loop

forward control integral has obvious effect and can filter low frequency

factor 1 interference, but the dynamic response is slow.

By adjusting this coefficient, the speed loop can have better

21Shenzhen Just Motion Control Electromechanics Co.,ltd

0755-26509689

dynamic response and increase the resistance of low frequency

interference.

Setting range: 1.00-2000.0, unit: Hz

The larger the speed proportional gain, the larger the servo

P02-13

Speed stiffness, the faster the speed response, but too large is easy to

proportional gain generate vibration and noise.

2 This parameter value is increased as far as possible under the

condition that the system is not oscillating.

This parameter is for dynamic response.

Setting range: 1.0-1000.0, unit: ms

Speed regulator integral time constant. The smaller the setting

P02-14

The speed value is, the faster the integral velocity is and the higher the

integral constant stiffness is.

2

This parameter value is minimized when the system does not

oscillate.

This parameter is for dynamic response.

Setting range: 0-100.0, unit: 1.0%

Pseudo

differential

P02-15

feedforward

control

coefficient 2

When the speed loop is set to 100.0%, PI control is adopted and

the dynamic response is fast. When set to 0, the velocity loop

integral has obvious effect and can filter low frequency

interference, but the dynamic response is slow.

By adjusting this coefficient, the speed loop can have better

dynamic response and increase the resistance of low frequency

interference.

Setting range: 0-30000, unit: 1.0%

P02-19

Torque feed Set the weighting value of current loop feedforward. The

forward gain parameter adds the current loop after weighting the differential

of the speed instruction

P02-20

Torque feed

Setting range: 0-64.00, unit: ms

22Shenzhen Just Motion Control Electromechanics Co.,ltd

0755-26509689

forward This parameter is used to set the torque feed forward filter time

smoothing constant.

constant

Setting range: 0-10

Set the first and second gain switching conditions

Valu Switching

e conditions

0 Fixed for

Remark

P02-00、P02-10、P02-11、P02-12

the first

gain

1

Fixed for

P02-01、P02-13、P02-14、P02-15

the second

gain

2

P02-30

use

Need to set the DI port to 9 (gain

Gain switching

switching input)

mode

DI input

switching

Invalid: first gain Effective: second

gain

3

Torque Switch to second gain when the

command torque instruction is greater than the

is big threshold (determined by P02-31 and

P02-32). Less than the threshold and

when it exceeds the P02-33 delay

setting, switch to the first gain.

4

The speed Switches to the second gain when the

instruction speed instruction changes more than

changes a the threshold (determined by P02-31

lot. and P02-32). Less than the threshold

and when it exceeds the P02-33 delay

23Shenzhen Just Motion Control Electromechanics Co.,ltd

0755-26509689

setting, switch to the first gain.

5

peed Switch to second gain when the speed

command instruction is greater than the

is large. threshold (determined by P02-31 and

P02-32). Switch to first gain when

less than threshold, while exceeding

P02-33 latency settings

6

Large Switch to second gain when the

position position deviation is greater than the

deviation threshold (determined by P02-31 and

P02-32). Less than the threshold and

when it exceeds the P02-33 delay

setting, switch to the first gain.

7

Have Switch to second gain when there is a

Location position command. The position

command. command ends and when it exceeds

the P02-33 delay setting, switch to the

first gain.

8

Location Switch to second gain when

incomplet positioning is not complete. Location

e. complete, while exceeding the P02-33

delay setting, switch to first gain

9

Real Switches to the second gain when the

speed is actual speed is greater than the

big threshold (determined by P02-31 and

P02-32). Less than the threshold and

when it exceeds the P02-33 delay

setting, switch to the first gain.

10

Location Switch to second gain when there is a

command position command. There is no

+ actual position instruction and the actual

24Shenzhen Just Motion Control Electromechanics Co.,ltd

0755-26509689

speed

speed is less than the threshold

(determined by P02-31 and P02-32),

and when the P02-33 delay setting is

exceeded, switch to the first gain.

Set range: 0-20000

Judgment threshold for gain switching.

P02-31

Gain switching level

Torque unit: 1000 bit = 25 % rated torque

Speed unit: 1000 bit = 200 rpm

Location Unit: 131072 bit per lap

Set range: 0-20000

P02-32

Gain switching

hysteresis

The hysteresis level during gain switching

Torque unit: 1000 bit = 25 % rated torque

Speed unit: 1000 bit = 200 rpm

Location Unit: 131072 bit per lap

P02-33

P02-34

Gain switching

delay

Position gain

switching time

Set range: 0-1000 .0, unit: MS

When switching from the second gain to the first gain, the time

from the trigger condition to the actual switch is satisfied.

Set range :0-1000.0,Unit:ms

Position Control Gain 1 Smooth Switch to Position Control Gain

2

Set range: 0-20000

P02-41

Mode Switch

Level

Set the threshold for switching.

Torque unit: 1000 bit = 25 % rated torque

Speed unit: 1000 bit = 200 rpm

Location Unit: 131072 bit per lap

Torque

P02-50

instruction plus

calculation

P02-51

Set range: -100.0-100, Unit 1.0 %

Location control mode is valid. This value is superimposed on a

given torque value and is used for static torque compensation of

the vertical axis.

Forward torque Set range: -100.0-100 .,Unit1.0 %

compensation Location control mode is valid. Used to compensate for forward

25Shenzhen Just Motion Control Electromechanics Co.,ltd

0755-26509689

static friction

P02-52

Reverse torque

compensation

Set range: -100.0-100 .0, Unit 1.0 %

Location control mode is valid. Used to compensate for reverse

static friction

5.2.4 P03-xx Position parameters

Paramet

er code

P03-00

P03-03

name Instruction

Location 0: Pulse command

Command Source 1: Numbers given, used for communication control.

Inverse command

pulse

Used to adjust pulse count direction

0: Normal.

1: Reverse direction

Setting range :0-3,UNIT:us

P03-04

Location Pulse

Filter Settings

0:0.1us。

1:0.4us

2:0.8us。

3:1.6us

Location

P03-05

complete

judgment

conditions

0: Output when position deviation is less than P03-06 setting

1: The position is given and the position deviation is less than

the P03-06 setting output

2: Location given(filtered), and position deviation less than

P03-06 set output

Set range: 0-65535, units: encoder units

P03-06

Location Use to set the threshold value for positioning completion output.

complete range Using the incremental encoder motor, the number of encoder

lines * 4 per loop is calculated.

P03-09

Number of Set range: 0-65535

command pulses Used to set the number of command pulses for the motor to

for 1 motor rotate around. When this parameter is set to 0, the P03-10 and

rotation P03-11 parameters are valid.

26Shenzhen Just Motion Control Electromechanics Co.,ltd

0755-26509689

P03-10

Molecular of The formula for calculating the electronic gear ratio of

electronic gear 1 =

G =

incremental

Deno min ator

P

eg : Encoderlin equantity 2500 ;Inputpuls equantityp errotation 3200 ;Electroni

C ::Lineofe coder ; P ::Inputpu senumberpe rroationof

Motors:

C × 4 2500 × 4 10000 25

Numerator

G =

The denominator

P03-11

of electronic gear

P

=

3200

=

3200

C × 4

=

8

Remark

1

Setting range: 0-65535, unit: instruction unit *10

P03-15

Position Set the number of pulses that are allowed to deviate it will alarm

deviation too if beyond the set value.

large setting Example: set value is 20. When the following deviation exceeds

20*10, the driver will alarm AL.501 (the position deviation is

too large).

Position

P03-16

command Setting range: 1000, unit: ms

smoothing Set the time constant of the position instruction smoothing filter

constant

5.2.5 P04-xx Speed parameter

Paramet

er code

name

Instruction

0: external analog instruction

P04-00

Speed command 1: digital instruction (parameter setting)

source 2: digital instruction (communication)

3: internal multiple sets of instructions

P04-02

P04-05

Digital speed

given value

Set range: -6000-6000, units: rpm

When P04-00 is set to 1, P04-02 is the speed control setting

value

Overspeed alarm Set range: 0-6500, unit: rpm

value Set allowed maximum speed, exceeding set value will A.420

27Shenzhen Just Motion Control Electromechanics Co.,ltd

0755-26509689

speed alarm

P04-06

P04-07

Forward speed Set range: 0-6000 in rpm

limit Limit motor forward speed

Reverse speed Set range: 0-6000 in rpm

limit Limit motor reverse speed

Set range: 0-200 .0 in rpm

P04-10

Zero speed Set the zero speed exit limit, the motor speed below the

detection value threshold can output the "motor zero speed output" signal

through the output port

P04-14 acceleration time

P04-15 Deceleration time

Setting range: 0-10000, unit: 1ms/1000rpm

Set the acceleration for speed control

Setting range: 0-10000, unit: 1ms/1000rpm

Set the deceleration speed for speed control

5.2.6 P05-xx Torque parameters

Parame

ter

name

Instruction

code

Setting range: 0-300.0, unit: 1.0%

The forward output of the motor is limited, with 100 denoting

P05-10

Internal positive

torque limit value

1 times torque and 300 denoting 3 times torque

When the torque output reaches the limit value, the output

signal can be detected by the torque limit output on the DO

port

Setting range: 0-300.0, unit: 1.0%

Limited motor output, 100 represents 1 times torque, 300

P05-11

Internal torque represents 3 times torque

limit value When the torque output reaches the limit value, the output

signal can be detected by the torque limit output on the DO

port

5.2.7 P06-xx I/O Parameter

28Shenzhen Just Motion Control Electromechanics Co.,ltd

0755-26509689

Paramet

er code

Name

Instruction

Enable the output

P06-00

port to be effectively

Setting range: 0-1, factory setting: 1

level

P06-20

Alarm output port

effective level

Setting range: 0-1, factory setting: 1

Put in place the

P06-22

output port effective

Setting range: 0-1, factory setting: 1

level

5.2.8 P08-xx Advanced function parameters

Parame

ter

Name

Instruction

code

P08-19

Feedback speed Setting range: 0-25.00, unit: ms

low-pass filtering The feedback speed low-pass filter time constant can be set

constant to large when the motor is roaring during operation.

Setting range: 0-25.00, unit: ms

P08-20

Torque command Torque instruction filter time constant, when the motor

filter constant running in the squealing, the value can be appropriately set

to large.

Setting range: 0-100.0

P08-25

Disturbance torque

compensation gain

Gain coefficient of perturbation torque observation. The

higher the value, the stronger the anti-disturbance torque

capacity, but the motion noise may also increase。

The perturbation

P08-26

torque filter time

constant

Setting range: 0-25.00, unit: ms

The larger the value, the stronger the filtering effect, can

inhibit the motion noise. However, the effect of disturbance

torque is affected by the phase delay.

5.3 Monitor project list

29Shenzhen Just Motion Control Electromechanics Co.,ltd

0755-26509689

displaying

serial

Description

Display item

Unit

number

d00.C.PU

d01.F.PU

The sum of the

position commands

This parameter can monitor the number of

pulses sent by the user to the servo driver, so

as to confirm whether any pulse loss occurs

The sum of the This parameter can monitor the pulse number

position feedback of the servo motor feedback. The unit is the

pulses same as the user input unit

Instruction

unit

Instruction

unit

This parameter can monitor the pulse number

d02.E.PU

Number of position of position delay in the operation of the servo Instruction

deviation pulses system. The unit is the same as the user input unit

unit

d03.C.PE

The position is This parameter can monitor the number of given the sum of pulses sent by the user to the servo driver. Encoder

the pulses / Unit: when using absolute value motor, unit/

calculate at 131072bit for each turn. With the Instruction

Feedback pulse of incremental encoder motor, the number of unit

gantry motor encoder lines per lap is *4.

This parameter can monitor the pulse number

d04.F.PE

of the servo motor feedback. Encoder

Position feedback Unit: when using absolute value motor, unit /

pulse sum calculate at 131072bit for each turn. With the Instruction

incremental encoder motor, the number of unit

encoder lines per lap is *4.

This parameter can monitor the pulse number

d05.E.PE

Position deviation of position delay in the operation of the servo Encoder

pulse number system. unit/

/Gantry pulse

deviation

Instruction

Unit: when using absolute value motor,

calculate at 131072bit for each turn. With the

30

unitShenzhen Just Motion Control Electromechanics Co.,ltd

0755-26509689

incremental encoder motor, the number of

encoder lines per lap is *4.

d06.C.Fr

d07.C.SP

d08.F.SP

d09. C.tQ

d10. F.tQ

Pulse command This parameter can monitor the input

input frequency frequency of external pulse instruction

Speed control

rpm

instruction

Motor speed

Torque command

KPPS

This parameter can monitor the speed when

the servo motor is running

rpm

This parameter can monitor the torque when

the servo motor is running

Torque feedback This parameter can monitor the torque

value feedback when the servo motor is running

%

%

This parameter can monitor the servo motor's

d11.AG.L The average torque d12.PE.L Peak torque d13.oL Overload load rate occupancy of the servo motor for the past 10

Regenerative load This parameter can monitor the load rate of

factor regenerative resistance

%

average torque for the past 10 seconds

This parameter can monitor the peak torque

%

of the servo motor after power on

This parameter can monitor the load

%

seconds

d14.rG

%

This parameter can monitor the input port

state of CN1. The upper vertical bar

represents the high level (optocoupler cutoff),

d16.I.Io

Input IO state

and the lower horizontal bar represents the binary

low electro-optical coupling conduction). The system

corresponding relation with the input port is

that the operation panel corresponds to

di1-di4 from right to left

d17.o.Io

Output IO state

This parameter can monitor the output port binary

state of CN1. The upper vertical bar system

31Shenzhen Just Motion Control Electromechanics Co.,ltd

0755-26509689

represents the optical coupling lead, while the

lower vertical bar represents the optical

coupling cut-off. The corresponding relation

with the output port is that the operation

panel corresponds to do1-do3 from right to

left

This parameter can monitor the motor

d18.AnG

Machine Angle

mechanical Angle, rotation 1 turn is 360

0.1degree

degrees

d19.HAL

d20.ASS

d21.ASH

Motor UVW phase

sequence

This parameter can monitor the phase

sequence position of the incremental encoder

motor

Absolute value This parameter can monitor the feedback

encoder single coil value of the absolute encoder, rotating one

number circle as 0xffff

10/5000 This parameter can be used to monitor the

Absolute value number of turns of the absolute multi-coil

encoder multi-turn encoder motor

0-0xFFFF

numerical value

d22.J-L

d23.dcp

d24.Ath

d25.tiE

d26.1.Fr

d28.2.Fr

Inertia ratio

This parameter can monitor the real-time

%

inertia of the motor load

Main circuit This parameter can monitor the voltage value

voltage (dc value) of the main circuit

Driver temperature

V

This parameter can monitor drive degree

temperature centigrade

Cumulative This parameter can monitor the drive run

running time time in seconds

Resonance This parameter can monitor the resonant

frequency 1 frequency 1

Resonance This parameter can monitor the resonant

frequency 2 frequency 2

32

Second

Hz

HzShenzhen Just Motion Control Electromechanics Co.,ltd

0755-26509689

d30.Ai1

Analog quantity This parameter can monitor the speed loop's

command 1 input analog instruction (v-ref) input voltage value.

0.01V

voltage(V_REF)

d31.Ai2

Analog quantity This parameter can monitor the input voltage

command 2 input value of the analog instruction (t-ref) of the

voltage(T_REF) torque loop.

0.01V

6 、 Fault analysis and treatment

6.1 Fault alarm information table

Alarm type

A hardware

failure

The serial

Alarm content

number code

AL.051 EEPROM parameter anomaly

AL.052 Programmable logic configuration failures

AL.053 Initialization failed

AL.054 A system exception

AL.060 Product model selection fault

AL.061 Product matching fault

AL.062 Parameter storage failure

AL.063 Overcurrent detection

AL.064

The servo self - check found the output to ground short

circuit fault

AL.066 Servo unit control power supply low

AL.070 AD sampling fault 1

AL.071 Current sampling fault

AL.100 Parameter combination anomaly

AL.101 AI setting fault

AL.102 DI distribution fault

AL.103 DO allocation fault

AL.105 Error setting of electronic gear

AL.106 Abnormal output setting of frequency division pulse

33Shenzhen Just Motion Control Electromechanics Co.,ltd

0755-26509689

AL.110 The parameters should be reset

AL.120 Invalid alert for servo command

AL.401 Under voltage

AL.402 Over voltage

AL.410 Overload (instantaneous maximum load)

AL.411 Driver overload

AL.412 Motor overload (maximum continuous load)

AL.420 Over speed

AL.421 Out of control check out

AL.422 Speed fault

AL.425 AI sampling voltage is too high

AL.435 Impulse current limits resistance overload

AL.436 DB overload

AL.440 Radiator overheating

AL.441 Motor overheat fault

AL.500 Frequency division pulse output overspeed

AL.501 Excessive deviation of position

AL.502

Full closed - loop encoder location and motor location

deviation is too large

AL.505 P command input pulse exception

AL.550 Failure identification of inertia

AL.551 Return to origin timeout fault

AL.552 Angle to identify failed faults

AL.600 Short circuit fault of encoder output power

AL.610 Incremental decoder offline

Encoder AL.611 Incremental encoder Z signal loss

fault AL.620 Bus type encoder off line

AL.621 Read/write motor encoder EEPROM parameter abnormal

Warning

AL.622 Motor encoder EEPROM data verification error

AL.900 Excessive deviation of position

AL.901 The position deviation of the servo ON is too large

34Shenzhen Just Motion Control Electromechanics Co.,ltd

0755-26509689

AL.910 Motor overload

AL.912 Driver overload

AL.941 Parameter changes that require reconnection

AL.942 Write EEPROM frequent warning

AL.943 Serial communication exception

AL.950 Overpass warning

AL.971 Undervoltage warning

6.2 Fault alarm reason and disposal

AL.051:EEPROM parameter anomaly

Fault alarm reason

Server EEPROM data

Fault alarm check

Check the wiring

exception

Treatment measure

Correct wiring and recharge

If always present, change the

drive

AL.053:Failure to initialize

Fault alarm reason Fault alarm check Treatment measure

Main control failed to initiate Check the wiring If always present, change the

power on MCU Back to electricity drive

AL.063:Overcurrent detection

Fault alarm reason Fault alarm check Treatment measure

The power module of servo Is there a short circuit in Correct connection

unit has excessive current U,V,W connection If always present, change the

Is there a short circuit drive

between B1 and B3

AL.071:Current sampling fault

Fault alarm reason Fault alarm check

Abnormal sampling data of Is the connection correct

current sensor

Treatment measure

Correct connection

If always present, change the

drive

AL.100:Parameter combination anomaly

Fault alarm reason

Fault alarm check

35

Treatment measureShenzhen Just Motion Control Electromechanics Co.,ltd

0755-26509689

Parameter setting error

Check the parameters set Set the parameters correctly

(p03-07) If always occurs, do

parameter initialization

AL.102:DIistribution of the fault

Fault alarm reason Fault alarm check Treatment measure

At least two input ports have Check port input function to Set the parameters correctly

the same function selection select parameters The drive is reenergized

AL.103:DO Distribution of the fault

Fault alarm reason Fault alarm check Treatment measure

At least two output ports have Check port output function to Set the parameters correctly

the same function selection select parameters The drive is reenergized

parameters

AL.105:Electronic gear setting error

Fault alarm reason Fault alarm check Treatment measure

Error setting of electronic Check the setting parameters Set the electronic gear ratio

gear ratio of the electronic gear correctly

ratio.P03-10,P03-11

The output pulse of the gantry Check the number of Correctly set the number of

is set too small feedback pulses for one feedback pulses for the

rotation of the gantry motor: rotation of one function motor

p03-52 must be larger than in Longmen.

128

AL.110:The parameters should be reset

Fault alarm reason Fault alarm check Treatment measure

After setting the servo Re-power the driver Re-power the driver

Fault alarm reason Fault alarm check Treatment measure

The input voltage of the main Check whether the input Ensure correct wiring and use

circuit is lower than the rated wiring of the main loop is the correct voltage source or

parameters, it needs to be

reenergized to take effect

AL.401:Under voltage

36Shenzhen Just Motion Control Electromechanics Co.,ltd

0755-26509689

voltage or no input voltage

correct and what voltage is

series connection stabilizer

AL.402:Over voltage

Fault alarm reason Fault alarm check Treatment measure

The input voltage of the main Use the voltmeter to test Use the correct voltage source

circuit is higher than the rated whether the input voltage of or serial voltage stabilizer

voltage the main circuit is correct Driver hardware fault When it is determined that Please send it back to the

the input voltage is correct, dealer or the original factory

the alarm is still over voltage for inspection

The regenerative resistance is Verify that p00-30 is set to 0 Correct setting and external

not connected or the or 1 regenerative resistance

regenerative resistance is not

selected correctly

AL.410:Overload (instantaneous maximum load)

Fault alarm reason Fault alarm check Treatment measure

The machine is stuck when Check if mechanical Adjust mechanical structure

the motor starts connections are stuck Driver hardware failure Verify that the mechanical Please send it back to the

part is normal and still alarm dealer or the original factory

for inspection

AL.412:Motor overload (continuous maximum load)

Fault alarm reason Fault alarm check Treatment measure

Use continuously beyond the It can be monitored through Change the motor or reduce

rated load of the driver d13.ol in the monitoring the load

mode

Improper parameter setting of 1. Whether the mechanical 1. Adjust the control loop gain

control system system is installed 2. Set time for acceleration

2. The acceleration setting and deceleration to slow down

constant is too fast

37Shenzhen Just Motion Control Electromechanics Co.,ltd

0755-26509689

3. Whether the gain class

parameters are set correctly

Motor wiring error

Check U, V, W connection

Correct connection

AL.420:Over speed

Fault alarm cause Fault alarm check The disposal measures

The input speed command is Use a signal detector to check Adjust the frequency of the

too high if the input signal is normal input signal

Incorrect parameter setting for Check whether p04-05 (speed Set p04-05 correctly (alarm

overspeed determination alarm value) is set properly value for overspeed)

Fault alarm cause Fault alarm check treatment measure

Drive internal temperature Check the heat dissipation Improve the heat dissipation

higher than 95 °C condition of the drive condition of the drive. If the

AL.440:Radiator overheating

alarm occurs again, please

send the drive back to the

original factory for inspection

AL.501:Position error is too big

Fault alarm reason Fault alarm check The disposal measures

The location deviation is too Confirm the parameter setting Increase the p03-15 (position

large and the parameter of p03-15 (location deviation deviation is too large) setting

setting is too small is too large) value

The gain is set too small Verify that the gain class Reset the gain class

parameters are set properly parameters correctly

Internal torque limit setting is Confirm the internal torque Adjust the internal torque

too small limit value limit value correctly

Excessive external load Check external load

Reduce load or replace power

motor

AL.505:P Command input pulse exception

Fault alarm reason Fault alarm check The disposal measures

The pulse command The pulse frequency meter is Set the input pulse frequency

frequency is higher than the used to detect whether the correctly

38Shenzhen Just Motion Control Electromechanics Co.,ltd

0755-26509689

rated input frequency

input frequency is higher than

the rated input frequency

AL.610:Incremental decoder offline

Fault alarm reason Fault alarm check The disposal measures

Incremental encoder Hall U, Check the encoder wiring correct wiring

Hall V, Hall W signal

anomaly

AL943:6/5000

Serial communication exception

Fault alarm reason Fault alarm check The disposal measures

Serial communication Check the wiring Add a filter to the wire

interference Check the baud rate Reduce the baud rate of serial

The baud rate of serial port is parameter p00-21 for serial communication

set too high communication

7、

、 Control signal connection

7.1 Control signal single terminal common anode connection

39Shenzhen Just Motion Control Electromechanics Co.,ltd

0755-26509689

Note:

40Shenzhen Just Motion Control Electromechanics Co.,ltd

0755-26509689

VCC is compatible with 5V~24V.

The resistance R1 is connected to the control signal terminal, and the resistance value is 3~5K.

7.2 Control signal single terminal common cathode connection

Note:

VCC is compatible with 5V~24V.

The resistance R1 is connected to the control signal terminal, and the resistance value is 3~5K.

41Shenzhen Just Motion Control Electromechanics Co.,ltd

0755-26509689

7.3 Control signal differential connection mode

Note: VCC is compatible with 5V~24V.The resistance R1 is connected to the control signal

terminal, and the resistance value is 3~5K.

42Shenzhen Just Motion Control Electromechanics Co.,ltd

0755-26509689

7.4 232 Serial communication wiring diagram

Color connection for

Definition

Description

232 serial

communication lines

NC hang in the air

RX receiving end

Brown and white

GND Power- Blue

TX The sender Blue and white

VCC Power +

43Shenzhen Just Motion Control Electromechanics Co.,ltd

0755-26509689

7.5 Control signal sequence diagram

In order to avoid some wrong actions and deviations, PUL, DIR and ENA should meet certain

requirements, as shown in the figure below:

Remark:

:

(1) t1: ENA (enabling signal) should be at least 5 chi ahead of time, which is determined to be

high. It is generally recommended that ENA+ and ENA- be suspended.

(2) t2: DIR at least predates the PUL count edge 6, indicating that the state is high or low.

(3) t3: the pulse width shall not be less than 2.5 clear s.

(4) t4: the width of low level is not less than 2.5 clear s.

44Shenzhen Just Motion Control Electromechanics Co.,ltd

0755-26509689

7.6 Servo motor speed torque characteristic curve

iHSV42-40-05-24-XXX4/5000 Torque characteristics

iHSV42-40-07-24-XXX4/5000 Torque characteristics

45Shenzhen Just Motion Control Electromechanics Co.,ltd

0755-26509689

iHSV57-30-10-36-XXX4/5000 Torque characteristics

iHSV57-30-14-36-XXX4/5000 Torque characteristics

46Shenzhen Just Motion Control Electromechanics Co.,ltd

0755-26509689

iHSV57-30-18-36-XXX4/5000 Torque characteristics

iHSV60-30-20-36-XXX4/5000 Torque characteristics

47Shenzhen Just Motion Control Electromechanics Co.,ltd

0755-26509689

iHSV60-30-40-48-XXX4/5000 Torque characteristics

iHSV86-30-44-48-XXX4/5000 Torque characteristics

48Shenzhen Just Motion Control Electromechanics Co.,ltd

0755-26509689

iHSV86-30-66-72-XXX4/5000 Torque characteristics

8. Setting of dial code for subdivision

8.1Subdivision Settings

The subdivision Settings are as follows: when SW1, SW2, SW3 and SW4 are all set as on, the

user's customized subdivision is effective, and this value can be set by our company's servo

software.

Drawing codes

switch

SW1

SW2

SW3

SW4

Subdivision

Default on on on on

800 off on on on

1600 on off on on

3200 off off on on

6400 on on off on

12800 off on off on

25600 on off off on

51200 off off off on

1000 on on on off

49Shenzhen Just Motion Control Electromechanics Co.,ltd

0755-26509689

2000 off on on off

4000 on off on off

5000 off off on off

8000 on on off off

10000 off on off off

20000 on off off off

40000 off off off off

8.2 Input edge Settings

SW5 dial code switch sets input edge, off means rising edge and on means falling edge.

8.3 Logical direction setting

When SW6 dial code switch off or on, the direction of current motor motion can be changed, off

= CCW (forward), on=CW (reverse).

9、

、 Error alarm and LED flashing frequency

Number of red light

Alarm Description Treatment measures

2 Driver overcurrent Is there a short circuit in the motor

3 Drive position deviation Check if driver "deviation in place"

flashes

UVW line

exceeds set value parameter is set correctly

4 Drive encoder alarm Check whether the encoder wire is

7 Driver overload

properly connected

Check whether the motor UVW wire

is connected correctly

50Shenzhen Just Motion Control Electromechanics Co.,ltd

0755-26509689

10. Installation Dimensions

Mounting dimensions (unit:mm)

IHSV42-XX Mounting dimensions

Model Length L(mm)

iHSV42-40-05-24-XXX 84

iHSV42-40-07-24-XXX 110

shaft(mm)

24

51Shenzhen Just Motion Control Electromechanics Co.,ltd

0755-26509689

iHSV57-XX Mounting dimensions

Model No. Length L(mm) shaft(mm)

iHSV57-30-10-36-01-T-33-XXX 110

iHSV57-30-14-36-01-T-33-XXX 130

iHSV57-30-18-36-01-T-33-XXX 150

52

33Shenzhen Just Motion Control Electromechanics Co.,ltd

0755-26509689

iHSV60-XX Mounting dimensions

Model No.

Length L(mm)

diameter of Shaft length

Shaft S (mm) (mm)

iHSV60-30-20-36-XXX 110 11

iHSV60-30-20-36-03-XXX 130 14

iHSV60-30-40-36-XXX 110 14

53

30Shenzhen Just Motion Control Electromechanics Co.,ltd

0755-26509689

iHSV86-XX Mounting dimensions

Model Length L(mm)

iHSV86-30-44-48-XXX 162

iHSV86-30-66-72-XXX 189

Shaft(mm)

38

54Shenzhen Just Motion Control Electromechanics Co.,ltd

0755-26509689

11、

、 RFQ and analysis

11.1 Power lamp is not on

Check whether the power supply has input and whether the line connection is correct.

Whether the input voltage is too low.

High input voltage will burn out the servo drive motor。

11.2 power on the red light to alarm

Whether the input voltage of servo drive motor is too high or too low.

Whether the servo drive motor has pulse input all the time before power on, resulting in

overshoot alarm.

11.3 Run a small angle of rotation after the red light alarm

In the configuration parameters of the servo drive motor, whether the pole logarithm of the motor

and the number of lines of the encoder are matched (the pole logarithm is: 4, and the number of

lines of the encoder is: 1000).

If the pulse input speed is greater than the motor's rated speed, the position is out of tolerance.

11.4 The pulse does not rotate after input

Whether the connection of the pulse input terminal of the servo drive motor is reliable.

Whether the servo drive motor can be released, whether the energy signal has input.

The electronic gear ratio is too large.

55Shenzhen Just Motion Control Electromechanics Co.,ltd

0755-26509689

12、

、 iHSV42/57/60/86-XX Parameter modification steps

1、 Choose JmcServoPcControl Servo adjust software, Double-click to open the following image:

56Shenzhen Just Motion Control Electromechanics Co.,ltd

0755-26509689

2、In the popup dialog box, set the corresponding options and open at the point, as shown below:

3. If the communication is successful, the following figure shows:

57Shenzhen Just Motion Control Electromechanics Co.,ltd

0755-26509689

58Shenzhen Just Motion Control Electromechanics Co.,ltd

0755-26509689

Note: If the connection is not available, please confirm whether COM port is selected correctly,

whether the communication line is connected properly, and then reconnect according to the

above steps. Click the upper left option

,Then pop up the following window, then the internal parameters of the driver will be uploaded

automatically. After uploading, customers can change the parameters according to their needs.

59Shenzhen Just Motion Control Electromechanics Co.,ltd

0755-26509689

Note: P00-xx is parameters of the motor and drive . The factory has been set up, and will not be

changed by customers.

The following three steps are taken as follows: modify, download, upload, as shown in the

following figure:

Note: After setting the corresponding parameters in the settings, download the changed

parameters to the driver according to the download option, and then upload the parameters to the

interface to verify whether the parameters have changed.

60